Part 1

General

Section  1.1.   General

1.1.1. Application

1.1.1.1. Application

1)   This Part applies to all buildings covered in this Code. (See Article 1.1.1.1. of Division A.)

1.1.2. Objectives and Functional Statements

1.1.2.1. Attributions to Acceptable Solutions

1)   For the purpose of compliance with this Code as required in Clause 1.2.1.1.(1)(b) of Division A, the objectives and functional statements attributed to the acceptable solutions in Division B shall be the objectives and functional statements identified in Sections 2.5., 3.10., 4.5., 5.10., 6.10., 7.2., 8.3., 9.38, and 10.4. (See Note A-1.1.2.1.(1).)

1.1.3. Climatic and Seismic Data

1.1.3.1. Climatic and Seismic Values

1)   Except as provided in Sentences (2) and (4) and as required by Sentence 9.7.4.3.(2), the climatic and seismic values required for the design of buildings under this Code shall be in conformance with the values established by the authority having jurisdiction.

2)   Where they have not been established by the authority having jurisdiction, the climatic values required for the design of buildings shall be in conformance with Sentence (3) and the values listed in Appendix C. (See Note A-1.1.3.1.(2).)

3)   The outside winter design temperatures determined from Appendix C shall be those listed for the January 2.5% values. (See Note A-1.1.3.1.(3).)

4)   Where they have not been established by the authority having jurisdiction, the seismic values required for the design of buildings under Part 4 and Part 9 shall be in conformance with Appendix C. (See Note A-1.1.3.1.(4).)

5)   The outside summer design temperatures determined from Appendix C shall be those listed for the July 2.5% dry values.

1.1.3.2. Depth of Frost Penetration

1)   Depth of frost penetration shall be established on the basis of local experience.

1.1.4. Fire Safety Plan

1.1.4.1. Fire Safety Plan

1)   Where a fire safety plan is required, it shall conform to Section 2.8. of Division B of the  British Columbia Fire Code.

Section  1.2.   Terms and Abbreviations

1.2.1. Definitions of Words and Phrases

1.2.1.1. Non-defined Terms

1)   Words and phrases used in Division B that are not included in the list of definitions in Article 1.4.1.2. of Division A shall have the meanings that are commonly assigned to them in the context in which they are used, taking into account the specialized use of terms by the various trades and professions to which the terminology applies.

2)   Where objectives and functional statements are referred to in Division B, they shall be the objectives and functional statements described in Parts 2 and 3 of Division A.

3)   Where acceptable solutions are referred to in Division B, they shall be the provisions stated in Parts 3 to 10.

1.2.1.2. Defined Terms

1)   The words and terms in italics in Division B shall have the meanings assigned to them in Article 1.4.1.2. of Division A.

1.2.2. Symbols and Other Abbreviations

1.2.2.1. Symbols and Other Abbreviations

1)   The symbols and other abbreviations in Division B shall have the meanings assigned to them in Article 1.4.2.1. of Division A and Article 1.3.2.1.

Section  1.3.   Referenced Documents and Organizations

1.3.1. Referenced Documents

1.3.1.1. Effective Date

1)   Unless otherwise specified herein, the documents referenced in this Code shall include all amendments, revisions, reaffirmations, reapprovals, addenda and supplements effective to 15 July 2019.

1.3.1.2. Applicable Editions

1)   Where documents are referenced in this Code, they shall be the editions designated in Table 1.3.1.2.

Table 1.3.1.2.

Documents Referenced in Book I (General) of the British Columbia Building Code(1)(2)

Forming Part of Sentence 1.3.1.2.(1)

| Issuing Agency | Document Number(3) | Title of Document | Code Reference |
| --- | --- | --- | --- |
| AAMA | 501-05 | Methods of Test for Exterior Walls | A-5.9.3. |
| AAMA | 501.1-05 | Standard Test Method for Water Penetration of Windows, Curtain Walls and Doors Using Dynamic Pressure | A-5.9.3. |
| AAMA | 501.2-09 | Quality Assurance and Diagnostic Water Leakage Field Check of Installed Storefronts, Curtain Walls, and Sloped Glazing Systems | A-5.9.3. |
| AAMA | 501.4-09 | Recommended Static Test Method for Evaluating Curtain Wall and Storefront Systems Subjected to Seismic and Wind-Induced Inter-Story Drifts | A-5.9.3. |
| AAMA | 501.5-07 | Test Method for Thermal Cycling of Exterior Walls | A-5.9.3. A-5.9.3.3.(1) |
| AAMA | 501.6-09 | Recommended Dynamic Test Method for Determining the Seismic Drift Causing Glass Fallout from a Wall System | A-4.1.8.18.(14) and (15) A-5.9.3. |
| AAMA | 1304-02 | Voluntary Specification for Forced Entry Resistance of Side-Hinged Door Systems | 9.7.5.2.(2) |
| ACGIH | 28th Edition | Industrial Ventilation: A Manual of Recommended Practice for Design | 6.2.1.1.(1) 6.3.2.14.(2) A-6.3.1.5. |
| ACI | 355.2-19 | Qualification of Post-Installed Mechanical Anchors in Concrete (ACI 355.2-19) and Commentary | 4.1.8.18.(7) |
| ACI | 355.4M-19 | Qualification of Post-Installed Adhesive Anchors in Concrete (ACI 355.4M-19) and Commentary | 4.1.8.18.(7) |
| AHRI | ANSI/AHRI 210/240-2008 | Performance Rating of Unitary Air-Conditioning and Air-Source Heat Pump Equipment | Table 9.36.3.10. |
| AHRI | 1060 (I-P/2013) | Performance Rating of Air-to-Air Exchangers for Energy Recovery Ventilation Equipment | 9.36.3.8.(4) |
| AHRI | ANSI/AHRI 1500 (2015) | Performance Rating of Commercial Space Heating Boilers | Table 9.36.3.10. |
| AISI | S201-12 | North American Standard for Cold-Formed Steel Framing - Product Data 2012 Edition | 9.24.1.2.(1) |
| ANSI | A135.6-2012 | Engineered Wood Siding | Table 5.9.1.1. 9.27.9.1.(1) |
| ANSI | A208.1-2009 | Particleboard | 9.23.15.2.(3) 9.29.9.1.(1) 9.30.2.2.(1) |
| ANSI/CSA | ANSI Z21.10.3-2017/CSA 4.3-2017 | Gas-fired water heaters, volume III, storage water heaters with input ratings above 75,000 Btu per hour, circulating and instantaneous | Table 9.36.4.2. |
| ANSI/CSA | ANSI Z21.47-2016/CSA 2.3-2016 | Gas-fired central furnaces | Table 9.36.3.10. |
| ANSI/CSA | ANSI Z21.50-2016/CSA 2.22-2016 | Vented decorative gas appliances | Table 9.36.3.10. |
| ANSI/CSA | ANSI Z21.56-2017/CSA 4.7-2017 | Gas-fired pool heaters | Table 9.36.4.2. |
| ANSI/CSA | ANSI Z83.8-2016/CSA 2.6-2016 | Gas unit heaters, gas packaged heaters, gas utility heaters and gas-fired duct furnaces | Table 9.36.3.10. |
| APA | ANSI/APA PRG 320-2018 | Standard for Performance-Rated Cross-Laminated Timber | 3.1.6.3.(3) |
| ASCE | ASCE/SEI (7-10) | Minimum Design Loads for Buildings and Other Structures | A-4.1.8.18.(14) and (15) A-9.4.2.1. and 9.4.2.2. |
| ASCE | ASCE/SEI (8-02) | Specification for the Design of Cold-Formed Stainless Steel Structural Members | A-4.3.4.2.(1) |
| ASCE | ASCE/SEI (49-12) | Wind Tunnel Testing for Buildings and Other Structures | 4.1.7.14.(1) |
| ASHRAE | 1997 | ASHRAE Handbook – Fundamentals | A-9.32.3.11. |
| ASHRAE | 2011 | ASHRAE Handbook – HVAC Applications | A-2.4.2.1.(1) |
| ASHRAE | 2013 | ASHRAE Handbook – Fundamentals | A-9.36.2.4.(1) Table A-9.36.2.4.(1)-D |
| ASHRAE | Guideline 12-2000 | Minimizing the Risk of Legionellosis Associated with Building Water Systems | 6.2.1.1.(1) 6.3.2.15.(9) 6.3.2.16.(1) |
| ASHRAE | ANSI/ASHRAE 62-2001 | Ventilation for Acceptable Indoor Air Quality (except Addendum n) | A-9.25.5.2. |
| ASHRAE | ANSI/ASHRAE 62.1-2016 | Ventilation for Acceptable Indoor Air Quality | 6.3.1.1.(2) 6.3.1.1.(3) 6.3.2.2.(1) |
| ASHRAE | ANSI/ASHRAE/IES 90.1-2019 | Energy Standard for Buildings Except Low-Rise Residential Buildings | 10.2.2.1.(1) A-10.2.2.2. |
| ASHRAE | ANSI/ASHRAE 140-2011 | Standard Method of Test for the Evaluation of Building Energy Analysis Computer Programs | 9.36.5.4.(8) 9.36.6.4.(1) |
| ASHRAE | ANSI/ASHRAE 188-2015 | Legionellosis: Risk Management for Building Water Systems | A-6.2.1.1. |
| ASME | B18.6.1-1981 | Wood Screws (Inch Series) | Table 5.9.1.1. 9.23.3.1.(3) A-9.23.3.1.(3) |
| ASME/CSA | ASME A17.1-2016/CSA B44-16 | Safety Code for Elevators and Escalators | 3.2.6.7.(2) 3.5.2.1.(1) 3.5.2.1.(2) 3.5.2.1.(3) 3.5.4.1.(2) 3.5.4.2.(1) 3.8.3.7.(1) A-3.5.2.1.(1) Table 4.1.5.11. Table 4.1.8.18. |
| ASTM | A123/A123M-17 | Standard Specification for Zinc (Hot-Dip Galvanized) Coatings on Iron and Steel Products | Table 5.9.1.1. Table 9.20.16.1. |
| ASTM | A153/A153M-16a | Standard Specification for Zinc Coating (Hot-Dip) on Iron and Steel Hardware | Table 5.9.1.1. Table 9.20.16.1. 9.23.2.4.(2) |
| ASTM | A252-10 | Standard Specification for Welded and Seamless Steel Pipe Piles | 4.2.3.8.(1) |
| ASTM | A283/A283M-18 | Standard Specification for Low and Intermediate Tensile Strength Carbon Steel Plates | 4.2.3.8.(1) |
| ASTM | A390-06 | Standard Specification for Zinc-Coated (Galvanized) Steel Poultry Fence Fabric (Hexagonal and Straight Line) | Table 9.10.3.1.-B |
| ASTM | A653/A653M-18 | Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process | Table 5.9.1.1. 9.3.3.2.(1) 9.23.2.4.(1) |
| ASTM | A792/A792M-10 | Standard Specification for Steel Sheet, 55% Aluminum-Zinc Alloy-Coated by the Hot-Dip Process | 9.3.3.2.(1) |
| ASTM | A1008/A1008M-18 | Standard Specification for Steel, Sheet, Cold-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, Solution Hardened, and Bake Hardenable | 4.2.3.8.(1) |
| ASTM | A1011/A1011M-18a | Standard Specification for Steel, Sheet and Strip, Hot-Rolled, Carbon, Structural, High-Strength Low-Alloy, High-Strength Low-Alloy with Improved Formability, and Ultra-High Strength | 4.2.3.8.(1) |
| ASTM | C4-04 | Standard Specification for Clay Drain Tile and Perforated Clay Drain Tile | Table 5.9.1.1. 9.14.3.1.(1) |
| ASTM | C27-98 | Standard Classification of Fireclay and High-Alumina Refractory Brick | 9.21.3.4.(1) |
| ASTM | C73-17 | Standard Specification for Calcium Silicate Brick (Sand-Lime Brick) | Table 5.9.1.1. 9.20.2.1.(1) |
| ASTM | C126-13 | Ceramic Glazed Structural Clay Facing Tile, Facing Brick, and Solid Masonry Units | Table 5.9.1.1. 9.20.2.1.(1) |
| ASTM | C177-19 | Standard Test Method for Steady-State Heat Flux Measurements and Thermal Transmission Properties by Means of the Guarded-Hot-Plate Apparatus | 9.36.2.2.(1) |
| ASTM | C212-17 | Standard Specification for Structural Clay Facing Tile | Table 5.9.1.1. 9.20.2.1.(1) |
| ASTM | C260/C260M-10a | Standard Specification for Air-Entraining Admixtures for Concrete | 9.3.1.8.(1) |
| ASTM | C411-19 | Standard Specification for Hot-Surface Performance of High-Temperature Thermal Insulation | 3.6.5.4.(4) 3.6.5.5.(1) 9.33.6.4.(4) 9.33.8.2.(2) |
| ASTM | C412M-15 | Standard Specification for Concrete Drain Tile | Table 5.9.1.1. 9.14.3.1.(1) |
| ASTM | C444M-17 | Standard Specification for Perforated Concrete Pipe | Table 5.9.1.1. 9.14.3.1.(1) |
| ASTM | C494/C494M-17 | Standard Specification for Chemical Admixtures for Concrete | 9.3.1.8.(1) |
| ASTM | C516-08e1 | Standard Specification for Vermiculite Loose Fill Thermal Insulation | A-9.25.2.4.(5) |
| ASTM | C518-17 | Standard Test Method for Steady-State Thermal Transmission Properties by Means of the Heat Flow Meter Apparatus | 9.36.2.2.(1) |
| ASTM | C553-13 | Standard Specification for Mineral Fiber Blanket Thermal Insulation for Commercial and Industrial Applications | Table 5.9.1.1. |
| ASTM | C612-14 | Standard Specification for Mineral Fiber Block and Board Thermal Insulation | Table 5.9.1.1. |
| ASTM | C700-18 | Standard Specification for Vitrified Clay Pipe, Extra Strength, Standard Strength, and Perforated | Table 5.9.1.1. 9.14.3.1.(1) |
| ASTM | C726-17 | Standard Specification for Mineral Wool Roof Insulation Board | Table 5.9.1.1. 9.25.2.2.(1) |
| ASTM | C754-18 | Standard Specification for Installation of Steel Framing Members to Receive Screw-Attached Gypsum Panel Products | Table A-9.11.1.4.-A Table A-9.11.1.4.-B Table A-9.11.1.4.-C Table A-9.11.1.4.-D |
| ASTM | C834-17 | Standard Specification for Latex Sealants | Table 5.9.1.1. 9.27.4.2.(2) |
| ASTM | C840-18b | Standard Specification for Application and Finishing of Gypsum Board | 3.1.6.6.(2) Table 5.9.1.1. 9.29.5.1.(3) A-9.29.5.1.(3) |
| ASTM | C920-18 | Standard Specification for Elastomeric Joint Sealants | Table 5.9.1.1. 9.27.4.2.(2) |
| ASTM | C954-18 | Standard Specification for Steel Drill Screws for the Application of Gypsum Panel Products or Metal Plaster Bases to Steel Studs from 0.033 in. (0.84 mm) to 0.112 in. (2.84 mm) in Thickness | 9.24.1.4.(1) |
| ASTM | C991-16 | Standard Specification for Flexible Fibrous Glass Insulation for Metal Buildings | Table 5.9.1.1. |
| ASTM | C1002-07 | Standard Specification for Steel Self-Piercing Tapping Screws for the Application of Gypsum Panel Products or Metal Plaster Bases to Wood Studs or Steel Studs | Table 5.9.1.1. 9.24.1.4.(1) 9.29.5.7.(1) |
| ASTM | C1055-03 | Standard Guide for Heated System Surface Conditions that Produce Contact Burn Injuries | A-6.5.1.1.(3) |
| ASTM | C1177/C1177M-17 | Standard Specification for Glass Mat Gypsum Substrate for Use as Sheathing | 3.1.5.14.(6) 3.1.5.15.(4) Table 5.9.1.1. Table 9.23.17.2.-A A-9.27.14.2.(2)(a) |
| ASTM | C1178/C1178M-18 | Standard Specification for Coated Glass Mat Water-Resistant Gypsum Backing Panel | 3.1.5.14.(6) 3.1.5.15.(4) Table 5.9.1.1. 9.29.5.2.(1) |
| ASTM | C1184-18e1 | Standard Specification for Structural Silicone Sealants | Table 5.9.1.1. 9.27.4.2.(2) |
| ASTM | C1193-16 | Standard Specification for Use of Joint Sealants | A-Table 5.9.1.1. A-9.27.4.2.(1) |
| ASTM | C1280-13 | Standard Specification for Application of Exterior Gypsum Panel Products for Use as Sheathing | Table 5.9.1.1. |
| ASTM | C1299-03 | Standard Guide for Use in Selection of Liquid-Applied Sealants | A-9.27.4.2.(1) |
| ASTM | C1311-14 | Standard Specification for Solvent Release Sealants | Table 5.9.1.1. 9.27.4.2.(2) |
| ASTM | C1330-18 | Standard Specification for Cylindrical Sealant Backing for Use with Cold Liquid-Applied Sealants | Table 5.9.1.1. 9.27.4.2.(3) |
| ASTM | C1363-11 | Standard Test Method for Thermal Performance of Building Materials and Envelope Assemblies by Means of a Hot Box Apparatus | A-5.9.4.1.(1) 9.36.2.2.(4) |
| ASTM | C1396/C1396M-17 | Standard Specification for Gypsum Board | 3.1.5.14.(6) 3.1.5.15.(4) 3.1.6.6.(2) 3.1.6.15.(1) Table 5.9.1.1. Table 9.23.17.2.-A 9.29.5.2.(1) Table 9.29.5.3. |
| ASTM | C1472-16 | Standard Guide for Calculating Movement and Other Effects When Establishing Sealant Joint Width | A-Table 5.9.1.1. A-9.27.4.2.(1) |
| ASTM | C1658/C1658M-18 | Standard Specification for Glass Mat Gypsum Panels | 3.1.5.14.(6) Table 5.9.1.1. |
| ASTM | D323-15a | Standard Test Method for Vapor Pressure of Petroleum Products (Reid Method) | 1.4.1.2.(1)(4) |
| ASTM | D1037-12 | Standard Test Methods for Evaluating Properties of Wood-Base Fiber and Particle Panel Materials | A-9.23.15.2.(4) |
| ASTM | D1143/D1143M-07 | Standard Test Methods for Deep Foundations Under Static Axial Compressive Load | A-4.2.7.2.(2) |
| ASTM | D1227/D1227M-13 | Standard Specification for Emulsified Asphalt Used as a Protective Coating for Roofing | Table 5.9.1.1. 9.13.2.2.(2) 9.13.3.2.(2) |
| ASTM | D1761-12 | Standard Test Methods for Mechanical Fasteners in Wood and Wood-Based Materials | A-9.27.5.4.(2) |
| ASTM | D2178/D2178M-13a | Standard Specification for Asphalt Glass Felt Used in Roofing and Waterproofing | Table 5.9.1.1. |
| ASTM | D2898-10 | Standard Practice for Accelerated Weathering of Fire-Retardant-Treated Wood for Fire Testing | 3.1.4.8.(3) 3.1.5.5.(3) 3.1.5.24.(1) 3.1.6.9.(6) 3.2.3.7.(4) 9.10.14.5.(3) 9.10.15.5.(3) |
| ASTM | D3019/D3019M-17 | Standard Specification for Lap Cement Used with Asphalt Roll Roofing, Non-Fibered, and Fibered | Table 5.9.1.1. 9.13.3.2.(2) Table 9.26.2.1.-B |
| ASTM | D3679-17 | Standard Specification for Rigid Poly (Vinyl Chloride) (PVC) Siding | 9.27.12.1.(1) |
| ASTM | D4477-16 | Standard Specification for Rigid (Unplasticized) Poly(Vinyl Chloride) (PVC) Soffit | 9.27.12.1.(3) |
| ASTM | D4479/D4479M-07e1 | Standard Specification for Asphalt Roof Coatings – Asbestos-Free | Table 5.9.1.1. 9.13.2.2.(2) 9.13.3.2.(2) Table 9.26.2.1.-B |
| ASTM | D4637/D4637M-15 | Standard Specification for EPDM Sheet Used In Single-Ply Roof Membrane | Table 5.9.1.1. 9.13.3.2.(2) Table 9.26.2.1.-B |
| ASTM | D4811/D4811M-16 | Standard Specification for Nonvulcanized (Uncured) Rubber Sheet Used as Roof Flashing | Table 5.9.1.1. 9.13.3.2.(2) Table 9.26.2.1.-B |
| ASTM | D5456-19 | Standard Specification for Evaluation of Structural Composite Lumber Products | 3.1.11.7.(5) |
| ASTM | D6878/D6878M-11a | Standard Specification for Thermoplastic Polyolefin Based Sheet Roofing | Table 5.9.1.1. 9.13.3.2.(2) Table 9.26.2.1.-B |
| ASTM | D7254-17 | Standard Specification for Polypropylene (PP) Siding | 9.27.13.1.(1) |
| ASTM | D7793-17 | Standard Specification for Insulated Vinyl Siding | 9.27.12.1.(2) |
| ASTM | D8052/D8052M-17 | Standard Test Method for Quantification of Air Leakage in Low-Sloped Membrane Roof Assemblies | A-5.4.1.2.(1) |
| ASTM | E90-09 | Standard Test Method for Laboratory Measurement of Airborne Sound Transmission Loss of Building Partitions and Elements | 5.8.1.2.(1) 5.8.1.4.(1) 9.11.1.2.(1) |
| ASTM | E96/E96M-16 | Standard Test Methods for Water Vapor Transmission of Materials | 5.5.1.2.(3) 9.13.2.2.(2) 9.25.4.2.(1) 9.25.4.2.(2) 9.25.5.1.(1) 9.30.1.2.(1) |
| ASTM | E283-04 | Standard Test Method for Determining Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen | 5.9.3.4.(2) A-5.9.3.4.(2) |
| ASTM | E330/E330M-14 | Standard Test Method for Structural Performance of Exterior Windows, Doors, Skylights and Curtain Walls by Uniform Static Air Pressure Difference | A-5.9.3.2.(1) |
| ASTM | E331-00 | Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Uniform Static Air Pressure Difference | 5.9.3.5.(2) A-5.9.3.5.(2) |
| ASTM | E336-11 | Standard Test Method for Measurement of Airborne Sound Attenuation between Rooms in Buildings | 5.8.1.2.(2) 5.8.1.4.(7) 9.11.1.2.(2) A-9.11. |
| ASTM | E413-16 | Classification for Rating Sound Insulation | A-1.4.1.2.(1)(4) 5.8.1.2.(1) 5.8.1.2.(2) 5.8.1.4.(7) 5.8.1.5.(3) 9.11.1.2.(1) 9.11.1.2.(2) |
| ASTM | E492-09e1 | Standard Test Method for Laboratory Measurement of Impact Sound Transmission Through Floor-Ceiling Assemblies Using the Tapping Machine | A-9.11. |
| ASTM | E547-00 | Standard Test Method for Water Penetration of Exterior Windows, Skylights, Doors, and Curtain Walls by Cyclic Static Air Pressure Difference | 5.9.3.5.(2) A-5.9.3.5.(2) |
| ASTM | E597-95 | Practice for Determining a Single Number Rating of Airborne Sound Insulation for Use in Multi-Unit Building Specifications | A-9.11. |
| ASTM | E736/E736M-17 | Standard Test Method for Cohesion/Adhesion of Sprayed Fire-Resistive Materials Applied to Structural Members | Table 9.10.3.1.-B |
| ASTM | E779-10 | Standard Test Method for Determining Air Leakage Rate by Fan Pressurization | 10.2.3.5.(1) |
| ASTM | E783-02 | Standard Test Method for Field Measurement of Air Leakage Through Installed Exterior Windows and Doors | A-5.4.1.2.(2) A-5.9.2.3.(1) A-5.9.3.4.(2) |
| ASTM | E1007-19 | Standard Test Method for Field Measurement of Tapping Machine Impact Sound Transmission Through Floor-Ceiling Assemblies and Associated Support Structures | A-9.11. |
| ASTM | E1105-15 | Standard Test Method for Field Determination of Water Penetration of Installed Exterior Windows, Skylights, Doors, and Curtain Walls, by Uniform or Cyclic Static Air Pressure Difference | A-5.9.2.3.(1) A-5.9.3.5.(2) |
| ASTM | E1186-17 | Standard Practices for Air Leakage Site Detection in Building Envelopes and Air Barrier Systems | A-5.4.1.2.(2) |
| ASTM | E1300-16 | Standard Practice for Determining Load Resistance of Glass in Buildings | 4.3.6.1.(1) 9.6.1.3.(1) |
| ASTM | E2190-19 | Standard Specification for Insulating Glass Unit Performance and Evaluation | Table 5.9.1.1. 9.6.1.2.(1) |
| ASTM | E2307-15b | Standard Test Method for Determining Fire Resistance of Perimeter Fire Barriers Using Intermediate-Scale, Multi-storey Test Apparatus | 3.1.8.3.(4) A-3.1.8.3.(2) 9.10.9.2.(4) |
| ASTM | E2357-18 | Standard Test Method for Determining Air Leakage Rate of Air Barrier Assemblies | A-5.4.1.1.(3) 9.36.2.9.(1) A-9.36.2.9.(1) |
| ASTM | E3158-18 | Standard Test Method for Measuring the Air Leakage Rate of a Large or Multizone Building | 10.2.3.5.(1) |
| ASTM | F476-14 | Standard Test Methods for Security of Swinging Door Assemblies | 9.7.5.2.(2) A-9.7.5.2.(2) |
| ASTM | F842-01 or 04 | Standard Test Methods for Measuring the Forced Entry Resistance of Sliding Door Assemblies, Excluding Glazing Impact | 9.7.5.1.(3) |
| ASTM | F1667-18a | Standard Specification for Driven Fasteners: Nails, Spikes, and Staples | 9.23.3.1.(1) 9.26.2.3.(1) 9.29.5.6.(1) |
| ASTM | F2090-17 | Standard Specification for Window Fall Prevention Devices With Emergency Escape (Egress) Release Mechanisms | A-9.8.8.1.(4) |
| ASTM | G115-10 | Standard Guide for Measuring and Reporting Friction Coefficients | 4.1.8.18.(18) |
| BC | 2023 | British Columbia Fire Code | 1.1.1.1.(1)(4) A-1.1.1.2.(1)(4) 1.1.4.1.(1) 1.4.1.2.(1)(4) 2.1.1.2.(4)(4) A-2.2.1.1.(1)(4) A-3.2.1.1.(1)(4) 3.1.13.1.(1) 3.2.3.21.(1) 3.2.5.16.(1) 3.3.1.2.(1) 3.3.1.10.(1) 3.3.2.3.(1) 3.3.2.16.(1) 3.3.4.3.(4) 3.3.5.2.(1) 3.3.6.1.(1) 3.3.6.3.(1) 3.3.6.3.(2) 3.3.6.4.(1) 3.3.6.4.(2) 3.3.6.6.(1) 3.7.3.1.(1) A-3.1.2.3.(1) A-3.2.4.6.(2) A-3.2.6. A-3.2.7.8.(3) A-3.3. A-3.3.1.7.(1) A-3.3.3.1.(1) A-3.3.6.1.(1) A-3.9.3.1.(1) 6.3.4.2.(3) 6.3.4.3.(1) 6.3.4.4.(1) 6.9.1.2.(1) 8.1.1.1.(3) 8.1.1.3.(1) 9.10.20.4.(1) 9.10.21.8.(1) |
| BC | 2023 | Book II (Plumbing Systems) of the British Columbia Building Code | 1.1.2.1.(1)(4) 2.1.1.2.(4)(4) A-2.2.1.1.(1)(4) A-3.2.1.1.(1)(4) A-4.1.6.4.(3) 5.6.2.2.(2) 6.3.2.15.(8) 6.3.2.15.(10) 6.3.2.16.(6) 7.1.2.1.(1) 7.1.2.1.(2) 7.1.2.1.(3) 7.1.2.1.(4) 9.31.6.2.(1) 9.36.3.11.(2) 9.36.4.3.(2) A-9.36.5.8.(5) Appendix C |
| BC | R.S.B.C. 1996 c.293 | Mines Act | 1.4.1.2.(1)(4) |
| BC | R.S.B.C. 2018 c.47 | Professional Governance Act | 1.4.1.2.(1)(4) |
| BC | S.B.C. 1998 c.43 | Strata Property Act | A-9.37.1.1. |
| BC | S.B.C. 2003 c.39 | Safety Standards Act | 3.2.6.7.(2) 3.3.6.2.(4) 3.5.2.1.(1) 3.5.2.1.(2) 3.5.4.1.(2) 3.6.1.2.(1) 3.6.2.1.(6) 3.6.2.7.(1) 6.2.1.5.(1) 6.2.1.5.(2) 9.10.22.1.(1) 9.31.6.2.(2) 9.33.5.2.(1) 9.34.1.1.(1) A-3.5.2.1.(1) Table 4.1.5.11. Table 4.1.8.18. |
| BC | S.B.C. 2015 | Building Act | 2.2.1.1.(1)(4) |
| BC | S.B.C. 2007 c.42 | Climate Change Accountability Act | 9.37.1.2.(2) 10.3.1.2.(1) |
| BNQ | BNQ 3624-115/2016 | Polyethylene (PE) Pipe and Fittings for Soil and Foundation Drainage | Table 5.9.1.1. 9.14.3.1.(1) |
| CCBFC | NRCC 35951 | Guidelines for Application of Part 3 of the National Building Code of Canada to Existing Buildings | A-1.1.1.1.(1)(4) |
| CCBFC | NRCC 38730 | Model National Energy Code of Canada for Houses 1997 | A-9.36.3.10.(1) A-9.36.4.2.(1) |
| CCBFC | NRCC 38732 | National Farm Building Code of Canada 1995 | 1.1.1.1.(3)(4) A-5.1.2.1.(1) |
| CCBFC | NRCC 40383 | User's Guide – NBC 1995, Fire Protection, Occupant Safety and Accessibility (Part 3) | A-1.1.1.1.(1)(4) |
| CCBFC | NRCC 43963 | User's Guide – NBC 1995, Application of Part 9 to Existing Buildings | A-1.1.1.1.(1)(4) |
| CCBFC | NRCC 56190 | National Building Code of Canada 2015 | A-4.1.8.4.(3) Appendix C |
| CCBFC | NRCC-CONST-56436E | National Plumbing Code of Canada 2020 | 7.1.2.1.(1) |
| CCBFC | NRCC-CONST-56437E | National Fire Code of Canada 2020 | 7.1.2.1.(2) |
| CCBFC | NRCC-CONST-56438E | National Energy Code of Canada for Buildings 2020 | A-2.1.1.2.(6)(4) A-2.2.1.1.(1)(4) A-3.2.1.1.(1)(4) A-5.4.1. A-2.2.8.1.(1)(5) 7.1.2.1.(2) 9.36.1.3.(1) 9.36.1.3.(4) 9.36.3.1.(2) Table 9.36.3.10. 9.36.4.1.(2) A-9.36.1.3. A-9.36.2.4.(1) A-9.36.3.10.(1) A-9.36.4.2.(1) A-9.36.5.2. 10.2.2.1.(1) 10.2.2.2.(2) 10.2.2.2.(3) 10.2.2.2.(4) Table 10.2.3.3.-A Table 10.2.3.3.-B 10.2.3.4.(1) 10.2.3.4.(4) A-10.2.2.2. A-10.2.3.3.(2) A-10.2.3.4. |
| CCBFC | NRCC-CONST-56529E | Structural Commentaries (User's Guide – NBC 2020: Part 4 of Division B) | A-1.1.1.1.(1)(4) A-4.1.1.3.(1) A-4.1.1.3.(2) A-4.1.2.1. A-4.1.2.1.(1) A-Table 4.1.2.1. A-4.1.3. A-4.1.3.2.(2) A-4.1.3.2.(4) A-4.1.3.2.(5) A-4.1.3.3.(2) A-4.1.3.4.(1) A-Table 4.1.3.4. A-4.1.3.5.(1) A-4.1.3.5.(3) A-4.1.3.6.(1) A-4.1.3.6.(2) A-4.1.3.6.(3) A-4.1.3.6.(4) A-4.1.5.5. A-4.1.5.8. A-4.1.5.17. A-4.1.6.1.(1) A-4.1.6.2. A-4.1.6.3.(2) A-4.1.6.4.(1) |
|  |  | Structural Commentaries (User's Guide – NBC 2020: Part 4 of Division B) (continued) | A-4.1.6.16. A-4.1.7.2.(2) A-4.1.7.3.(5)(c) A-4.1.7.3.(10) A-4.1.7.7.(2) A-4.1.7.9.(1) A-4.1.7.13. A-4.1.8.2.(1) A-4.1.8.3.(4) A-4.1.8.3.(6) A-4.1.8.3.(7)(b) and (c) A-4.1.8.3.(8) A-4.1.8.4.(2) and (3) A-4.1.8.4.(3) A-Table 4.1.8.5.-A A-Table 4.1.8.6. A-4.1.8.7.(1) A-4.1.8.9.(4) A-4.1.8.9.(5) A-4.1.8.10.(5) and (6) A-4.1.8.10.(7) A-4.1.8.10.(9) A-4.1.8.10.(10)(a) A-4.1.8.11.(3) A-4.1.8.12.(1)(a) A-4.1.8.12.(1)(b) A-4.1.8.12.(3) A-4.1.8.12.(4)(a) A-4.1.8.13.(4) A-4.1.8.15.(1) A-4.1.8.15.(3) A-4.1.8.15.(4) A-4.1.8.15.(5) A-4.1.8.15.(6) A-4.1.8.15.(7) A-4.1.8.15.(8) A-4.1.8.16.(1) A-4.1.8.16.(4) A-4.1.8.16.(6)(a) A-4.1.8.16.(7) A-4.1.8.16.(8)(a) A-4.1.8.16.(10) A-4.1.8.17.(1) A-4.1.8.18. A-4.1.8.18.(7)(e) A-4.1.8.18.(13) and 4.4.3.1.(1) A-4.1.8.18.(14) and (15) A-4.1.8.18.(16) A-4.1.8.19.(3)(a) A-4.1.8.19.(4) and 4.1.8.21.(5) A-4.1.8.21.(4)(a) A-4.2.4.1.(3) A-4.2.4.1.(5) A-4.2.5.1.(1) A-4.2.6.1.(1) A-4.2.7.2.(1) A-4.3.6.1.(1) A-4.4.2.1.(1) A-5.1.4.2. A-5.2.2.2.(4) Table C-3 |
| CGSB | CAN/CGSB-1.501-M89 | Method for Permeance of Coated Wallboard | 5.5.1.2.(2) 9.25.4.2.(7) |
| CGSB | CAN/CGSB-7.2-94 | Adjustable Steel Columns | 9.17.3.4.(1) A-9.17.3.4. |
| CGSB | CAN/CGSB-10.3-92 | Air Setting Refractory Mortar | 9.21.3.4.(2) 9.21.3.9.(1) 9.22.2.2.(2) |
| CGSB | CAN/CGSB-11.3-M87 | Hardboard | Table 5.9.1.1. 9.29.7.1.(1) 9.30.2.2.(1) |
| CGSB | CAN/CGSB-12.1-2017 | Safety Glazing | 3.3.1.20.(3) 3.3.2.17.(1) 3.3.2.17.(2) 3.4.6.15.(1) 3.4.6.15.(3) 3.7.2.4.(1) Table 5.9.1.1. 9.6.1.2.(1) 9.6.1.4.(1) 9.6.1.4.(6) 9.8.8.7.(1) |
| CGSB | CAN/CGSB-12.2-M91 | Flat, Clear Sheet Glass | Table 5.9.1.1. 9.6.1.2.(1) |
| CGSB | CAN/CGSB-12.3-M91 | Flat, Clear Float Glass | Table 5.9.1.1. 9.6.1.2.(1) |
| CGSB | CAN/CGSB-12.4-M91 | Heat Absorbing Glass | Table 5.9.1.1. 9.6.1.2.(1) |
| CGSB | CAN/CGSB-12.8-97 | Insulating glass units | Table 5.9.1.1. 9.6.1.2.(1) |
| CGSB | CAN/CGSB-12.9-M91 | Spandrel glass | Table 5.9.1.1. 9.6.1.2.(1) |
| CGSB | CAN/CGSB-12.10-M76 | Glass, Light and Heat Reflecting | 9.6.1.2.(1) |
| CGSB | CAN/CGSB-12.11-M90 | Wired Safety Glass | 3.3.1.20.(3) 3.4.6.15.(1) 3.4.6.15.(3) 9.6.1.2.(1) 9.6.1.4.(1) 9.8.8.7.(1) |
| CGSB | CAN/CGSB-12.20-M89 | Structural Design of Glass for Buildings | 4.3.6.1.(1) 9.6.1.3.(1) A-9.6.1.3.(2) |
| CGSB | CAN/CGSB-19.22-M89 | Mildew-Resistant Sealing Compound for Tubs and Tiles | 9.29.10.5.(1) |
| CGSB | 37-GP-9Ma-1983 | Primer, Asphalt, Unfilled, for Asphalt Roofing, Dampproofing and Waterproofing | Table 5.9.1.1. 9.13.3.2.(2) Table 9.26.2.1.-A |
| CGSB | CAN/CGSB-37.50-M89 | Hot-Applied, Rubberized Asphalt for Roofing and Waterproofing | Table 5.9.1.1. 9.13.3.2.(2) Table 9.26.2.1.-B |
| CGSB | CAN/CGSB-37.51-M90 | Application for Hot-Applied Rubberized Asphalt for Roofing and Waterproofing | 9.26.15.1.(1) |
| CGSB | CAN/CGSB-37.54-95 | Polyvinyl Chloride Roofing and Waterproofing Membrane | Table 5.9.1.1. 9.13.3.2.(2) Table 9.26.2.1.-B |
| CGSB | 37-GP-55M-1979 | Application of Sheet Applied Flexible Polyvinyl Chloride Roofing Membrane | 9.26.16.1.(1) |
| CGSB | 37-GP-56M-1985 | Membrane, Modified, Bituminous, Prefabricated, and Reinforced for Roofing | 9.13.3.2.(2) Table 9.26.2.1.-B |
| CGSB | CAN/CGSB-37.58-M86 | Membrane, Elastomeric, Cold-Applied Liquid, for Non-Exposed Use in Roofing and Waterproofing | Table 5.9.1.1. 9.13.3.2.(2) Table 9.26.2.1.-B |
| CGSB | CAN/CGSB-41.24-95 | Rigid Vinyl Siding, Soffits and Fascia | Table 5.9.1.1. |
| CGSB | CAN/CGSB-51.25-M87 | Thermal Insulation, Phenolic, Faced | Table 9.23.17.2.-A 9.25.2.2.(1) |
| CGSB | 51-GP-27M-1979 | Thermal Insulation, Polystyrene, Loose Fill | 9.25.2.2.(1) |
| CGSB | CAN/CGSB-51.32-M77 | Sheathing, Membrane, Breather Type | Table 5.9.1.1. 9.20.13.9.(1) Table 9.26.2.1.-A 9.27.3.2.(1) |
| CGSB | CAN/CGSB-51.33-M89 | Vapour Barrier Sheet, Excluding Polyethylene, for Use in Building Construction | Table 5.9.1.1. 9.25.4.2.(5) A-9.25.4.2.(6) |
| CGSB | CAN/CGSB-51.34-M86 | Vapour Barrier, Polyethylene Sheet for Use in Building Construction | Table 5.9.1.1. 9.13.2.2.(2) 9.18.6.2.(1) 9.25.3.2.(2) 9.25.3.6.(1) 9.25.4.2.(4) |
| CGSB | CAN/CGSB-51.71-2005 | Depressurization Test | 9.32.3.8.(7) |
| CGSB | CAN/CGSB-71.26-M88 | Adhesive for Field-Gluing Plywood to Lumber Framing for Floor Systems | A-9.23.4.2.(2) Table A-9.23.4.2.(2)-C |
| CGSB | CAN/CGSB-82.6-M86 | Doors, Mirrored Glass, Sliding or Folding, Wardrobe | 9.6.1.2.(2) A-9.6.1.2.(2) |
| CGSB | CAN/CGSB-93.1-M85 | Sheet, Aluminum Alloy, Prefinished, Residential | Table 5.9.1.1. 9.27.11.1.(3) A-9.27.11.1.(2) and (3) |
| CGSB | CAN/CGSB-93.2-M91 | Prefinished Aluminum Siding, Soffits, and Fascia, for Residential Use | 3.2.3.6.(5) Table 5.9.1.1. 9.10.14.5.(8) 9.10.14.5.(12) 9.10.15.5.(7) 9.10.15.5.(11) 9.27.11.1.(2) A-9.27.11.1.(2) and (3) |
| CGSB | CAN/CGSB-149.10-2019 | Determination of the airtightness of building envelopes by the fan depressurization method | 9.36.7.3.(1) |
| CISC/ICCA | 2018 | Crane-Supporting Steel Structures: Design Guide (Third Edition) | A-4.1.3.2.(2) |
| CMHC | 1988 | Air Permeance of Building Materials | Table A-9.25.5.1.(1) |
| CMHC | 1993 | Testing of Fresh Air Mixing Devices | A-9.32.3.4. |
| CoV | Version 2.0 | City of Vancouver Energy Modeling Guidelines | 10.2.3.4.(1) 10.2.3.4.(3) 10.2.3.4.(4) 10.3.1.1.(2) A-10.2.3.4. |
| CSA | 6.19-01 | Residential carbon monoxide alarming devices | 6.9.3.1.(2) 9.32.3.9.(2) 9.32.3.9.(3) |
| CSA | A23.1:19 | Concrete materials and methods of concrete construction | 4.2.3.6.(1) 4.2.3.9.(1) Table 5.9.1.1. 9.3.1.1.(1) 9.3.1.1.(4) 9.3.1.3.(1) 9.3.1.4.(1) |
| CSA | A23.3:19 | Design of concrete structures | Table 4.1.8.9. 4.1.8.18.(7) 4.3.3.1.(1) A-4.1.3.2.(4) A-4.1.8.16.(1) A-4.1.8.16.(4) A-4.3.3.1.(1) |
| CSA | A23.4-16 | Precast concrete – Materials and construction | A-4.3.3.1.(1) |
| CSA | CAN/CSA-A82:14 | Fired masonry brick made from clay or shale | Table 5.9.1.1. 9.20.2.1.(1) |
| CSA | CAN/CSA A82.27-M91 | Gypsum Board | 3.1.5.14.(6) 3.1.5.15.(4) 3.1.6.6.(2) 3.1.6.15.(1) |
| CSA | A82.30-M1980 | Interior Furring, Lathing and Gypsum Plastering | 9.29.4.1.(1) |
| CSA | A82.31-M1980 | Gypsum Board Application | 3.2.3.6.(5) 9.10.9.2.(5) 9.10.12.4.(3) 9.10.14.5.(8) 9.10.14.5.(12) 9.10.15.5.(7) 9.10.15.5.(11) 9.29.5.1.(2) Table 9.10.3.1.-A |
| CSA | CAN3-A93-M82 | Natural Airflow Ventilators for Buildings | Table 5.9.1.1. 9.19.1.2.(5) |
| CSA | CAN/CSA-A123.2-03 | Asphalt-Coated Roofing Sheets | Table 5.9.1.1. 9.13.3.2.(2) Table 9.26.2.1.-B |
| CSA | A123.3-05 | Asphalt Saturated Organic Roofing Felt | Table 5.9.1.1. Table 9.26.2.1.-B |
| CSA | CAN/CSA-A123.4-04 | Asphalt for Constructing Built-Up Roof Coverings and Waterproofing Systems | Table 5.9.1.1. 9.13.2.2.(2) 9.13.3.2.(2) Table 9.26.2.1.-B |
| CSA | A123.5:16 | Asphalt shingles made from glass felt and surfaced with mineral granules | Table 5.9.1.1. Table 9.26.2.1.-B |
| CSA | CAN/CSA-A123.16:04 | Asphalt-coated glass-base sheets | Table 5.9.1.1. Table 9.26.2.1.-B |
| CSA | A123.17-05 | Asphalt Glass Felt Used in Roofing and Waterproofing | Table 5.9.1.1. 9.13.3.2.(2) Table 9.26.2.1.-B |
| CSA | CAN/CSA-A123.21:14 | Standard test method for the dynamic wind uplift resistance of membrane-roofing systems | 5.2.2.2.(4) A-5.2.2.2.(4) |
| CSA | A123.22-08 | Self-Adhering Polymer Modified Bituminous Sheet Materials Used as Steep Roofing Underlayment for Ice Dam Protection | Table 9.26.2.1.-B |
| CSA | A123.23-15 | Product specification for polymer-modified bitumen sheet, prefabricated and reinforced | Table 5.9.1.1. Table 9.26.2.1.-B |
| CSA | A123.51-14 | Asphalt shingle application on roof slopes 1:6 and steeper | Table 5.9.1.1. 9.26.1.3.(1) |
| CSA | A165.1-14 | Concrete block masonry units | Table 5.9.1.1. 9.15.2.2.(1) 9.17.5.1.(1) 9.20.2.1.(1) 9.20.2.6.(1) Table A-9.11.1.4.-A Table A-9.11.1.4.-C |
| CSA | A165.2-14 | Concrete brick masonry units | Table 5.9.1.1. 9.20.2.1.(1) |
| CSA | A165.3-14 | Prefaced concrete masonry units | Table 5.9.1.1. 9.20.2.1.(1) |
| CSA | CAN/CSA-A179-14 | Mortar and Grout for Unit Masonry | Table 5.9.1.1. 9.15.2.2.(3) 9.20.3.1.(1) |
| CSA | CAN/CSA-A220 Series-06 | Concrete Roof Tiles | Table 5.9.1.1. Table 9.26.2.1.-B 9.26.17.1.(1) |
| CSA | A277-16 | Procedure for certification of prefabricated buildings, modules, and panels | A-1.1.1.1.(2)(4) |
| CSA | CAN/CSA-A324-M88 | Clay Flue Liners | 9.21.3.3.(1) |
| CSA | CSA-A344.2-05 | Standard for the design and construction of steel storage racks | 4.1.8.18.(13) |
| CSA | CAN/CSA-A370:14 | Connectors for masonry | A-9.21.4.5.(2) |
| CSA | CAN/CSA-A371-14 | Masonry Construction for Buildings | Table 5.9.1.1. 9.15.2.2.(3) 9.20.3.2.(7) 9.20.15.2.(1) |
| CSA | CAN/CSA-A405-M87 | Design and Construction of Masonry Chimneys and Fireplaces | 9.21.3.5.(1) 9.22.1.4.(1) 9.22.5.2.(2) |
| CSA | AAMA/WDMA/CSA 101/I.S.2/A440-17 | North American Fenestration Standard/Specification for windows, doors, and skylights | 5.9.2.2.(1) A-5.3.1.2. A-5.9.2.3.(1) A-5.9.3.1.(1) 9.7.4.3.(1) 9.7.4.3.(2) 9.7.5.1.(1) 9.7.5.3.(1) 9.36.2.9.(3) A-9.7.4.2.(1) |
| CSA | A440S1:19 | Canadian Supplement to AAMA/WDMA/CSA 101/I.S.2/A440-17, North American Fenestration Standard/Specification for windows, doors, and skylights | 5.9.2.2.(1) 5.9.3.5.(3) A-5.9.2.2. A-5.9.3.5.(3) 9.7.4.2.(1) 9.36.2.9.(3) A-9.7.4.2.(1) |
| CSA | A440.2:19/A440.3:19 | Fenestration energy performance/User guide to CSA A440.2:19, Fenestration energy performance | Table 9.7.3.3. 9.36.2.2.(3) A-Table 9.36.2.7.-A |
| CSA | A440.2:19 | Fenestration energy performance | A-5.3.1.2. A-5.9.3.3.(1) Table 9.36.8.6. A-9.7.4.2.(1) |
| CSA | A440.3:19 | User guide to CSA A440.2:19, Fenestration energy performance | A-5.3.1.2. |
| CSA | A440.4:19 | Window, door, and skylight installation | A-5.9.2.3.(1) |
| CSA | A660-10 | Certification of manufacturers of steel building systems | 4.3.4.3.(1) |
| CSA | A3001-18 | Cementitious Materials for Use in Concrete | Table 5.9.1.1. 9.3.1.2.(1) 9.28.2.1.(1) |
| CSA | B51:19 | Boiler, pressure vessel, and pressure piping code | 6.2.1.5.(1) 9.31.6.2.(2) 9.33.5.2.(1) |
| CSA | B52:18 | Mechanical refrigeration code | 6.2.1.5.(1) 9.33.5.2.(1) |
| CSA | B55.1:15 | Test method for measuring efficiency and pressure loss of drain water heat recovery units | 9.36.5.12.(2) |
| CSA | CAN/CSA-B72-M87 | Installation Code for Lightning Protection Systems | 3.6.1.3.(1) |
| CSA | B111-1974 | Wire Nails, Spikes and Staples | 9.23.3.1.(1) 9.26.2.3.(1) 9.29.5.6.(1) A-Table 9.23.3.5.-B |
| CSA | B139 Series:19 | Installation code for oil-burning equipment | 6.2.1.5.(1) 9.31.6.2.(2) 9.33.5.2.(1) |
| CSA | B140.4:04 | Oil-Fired Warm Air Furnaces | Table 9.36.3.10. |
| CSA | B140.12-03 | Oil-Burning Equipment: Service Water Heaters for Domestic Hot Water, Space Heating, and Swimming Pools | Table 9.36.4.2. |
| CSA | B149.1-15 | Natural gas and propane installation code | 6.2.1.5.(1) 9.10.22.1.(1) 9.31.6.2.(2) 9.33.5.2.(1) A-9.10.22. |
| CSA | ANSI/CSA-B149.6-15 | Code for digester gas, landfill gas, and biogas generation and utilization | 2.2.8.1.(3) |
| CSA | B182.1-18 | Plastic drain and sewer pipe and pipe fittings | Table 5.9.1.1. 9.14.3.1.(1) |
| CSA | CAN/CSA-B211-00 | Energy Efficiency of Oil-Fired Storage Tank Water Heaters | Table 9.36.4.2. |
| CSA | B214-16 | Installation code for hydronic heating systems | 6.2.1.1.(1) 9.33.4.2.(1) A-9.36.3.4.(1) |
| CSA | B355:19 | Platform lifts and stair lifts for barrier-free access | 3.8.3.7.(1) |
| CSA | B365-17 | Installation code for solid-fuel-burning appliances and equipment | 6.2.1.5.(1) 9.22.10.2.(1) 9.31.6.2.(2) 9.33.5.3.(1) A-9.33.1.1.(2) A-9.33.5.3. |
| CSA | B415.1-10 | Performance Testing of Solid-Fuel-Burning Heating Appliances | Table 9.36.3.10. |
| CSA | B651-18 | Accessible design for the built environment | 3.3.1.19.(1) 3.8.3.1.(1) Table 3.8.3.1. 3.8.3.3.(1) 3.8.3.9.(1) 3.8.3.9.(2) A-3.8.2.5. A-3.8.3.1.(1) |
| CSA | C22.1-18 | Canadian Electrical Code, Part I (24th edition), Safety Standard for Electrical Installations | A-3.1.4.3.(1)(b)(i) A-3.2.4.20.(9)(a) A-3.3.6.2.(4) A-9.10.22. A-9.34.2. A-9.35.2.2.(1) |
| CSA | C22.2 No. 0.3-09 | Test methods for electrical wires and cables | 3.1.4.3.(1) 3.1.4.3.(3) 3.1.5.21.(1) 3.1.5.21.(3) 9.34.1.5.(1) |
| CSA | C22.2 No. 113-10 | Fans and Ventilators | 9.32.3.10.(7) |
| CSA | C22.2 No. 141:15 | Emergency lighting equipment | 3.2.7.4.(2) 3.4.5.1.(3) 9.9.11.3.(3) 9.9.12.3.(7) |
| CSA | CAN/CSA-C22.2 No. 150-M89 | Microwave Ovens | A-9.10.22. |
| CSA | C22.2 No. 211.0-03 | General Requirements and Methods of Testing for Nonmetallic Conduit | 3.1.5.23.(1) |
| CSA | CAN/CSA-C22.2 No. 262-04 | Optical Fiber Cable and Communication Cable Raceway Systems | 3.1.5.23.(1) |
| CSA | CAN/CSA-C191-04 | Performance of Electric Storage Tank Water Heaters for Domestic Hot Water Service | Table 9.36.4.2. |
| CSA | CAN/CSA-C260-M90 | Rating the Performance of Residential Mechanical Ventilating Equipment | 9.32.3.10.(1) 9.32.3.10.(2) Table 9.32.3.10.-B |
| CSA | C282-15 | Emergency electrical power supply for buildings | 3.2.7.5.(1) |
| CSA | C368.1:14 | Energy performance of room air conditioners | Table 9.36.3.10. |
| CSA | CAN/CSA-C439-09 | Standard laboratory methods of test for rating the performance of heat/energy-recovery ventilators | 9.32.3.10.(4) 9.32.3.10.(5) 9.36.3.8.(4) 9.36.3.9.(3) A-9.36.3.9.(3) |
| CSA | CAN/CSA-C448 Series-13 | Design and installation of earth energy systems | 9.33.5.2.(1) |
| CSA | C656-14 | Performance standard for split-system and single-package air conditioners and heat pumps | Table 9.36.3.10. |
| CSA | CAN/CSA-C745-03 | Energy Efficiency of Electric Storage Tank Water Heaters and Heat Pump Water Heaters | Table 9.36.4.2. Table 9.36.8.10. |
| CSA | CAN/CSA-C746-06 | Performance Standard for Rating Large and Single Packaged Vertical Air Conditioners and Heat Pumps | Table 9.36.3.10. |
| CSA | C748-13 | Performance of direct-expansion (DX) ground-source heat pumps | Table 9.36.3.10. |
| CSA | CAN/CSA-C749-07 | Performance of Dehumidifiers | Table 9.36.3.10. |
| CSA | CAN/CSA-C828-13 | Performance requirements for thermostats used with individual room electric space heating devices | 9.36.3.6.(3) |
| CSA | CAN/CSA-C13256-1-01 | Water-Source Heat Pumps - Testing and Rating for Performance - Part 1: Water-to-Air and Brine-to-Air Heat Pumps (Adopted ISO 13256-1:1998, first edition, 1998-08-15, with Canadian Deviations) | Table 9.36.3.10. |
| CSA | CAN/CSA-C13256-2-01 | Water-Source Heat Pumps - Testing and Rating for Performance - Part 2: Water-to-Water and Brine-to-Water Heat Pumps (Adopted ISO 13256-2:1998, first edition, 1998-08-15, with Canadian Deviations) | Table 9.36.3.10. |
| CSA | F280-12 | Determining the required capacity of residential space heating and cooling appliances | 9.33.5.1.(1) A-9.36.3.2.(1) A-9.36.5.15.(5) |
| CSA | CAN/CSA-F326-M91 | Residential Mechanical Ventilation Systems | 9.32.3.1.(1) A-9.32.3.1.(1) A-9.32.3.5. A-9.32.3.7. A-9.32.3.8. A-9.33.6.13. |
| CSA | G30.18-09 | Carbon steel bars for concrete reinforcement | 9.3.1.1.(4) |
| CSA | G40.21-13 | Structural quality steel | 4.2.3.8.(1) Table 5.9.1.1. 9.23.4.3.(2) |
| CSA | CAN/CSA-G401-14 | Corrugated steel pipe products | Table 5.9.1.1. 9.14.3.1.(1) |
| CSA | CAN/CSA-O80 Series-15 | Wood preservation | 3.1.4.5.(1) 4.2.3.2.(1) Table 5.9.1.1. |
| CSA | CAN/CSA-O80.0-15 | General requirements for wood preservation | 4.2.3.2.(2) |
| CSA | CAN/CSA-O80.1-15 | Specification of treated wood | 4.2.3.2.(1) 9.3.2.9.(5) |
| CSA | CAN/CSA-O80.2-15 | Processing and treatment | 4.2.3.2.(1) |
| CSA | CAN/CSA-O80.3-15 | Preservative formulations | 4.2.3.2.(1) |
| CSA | O86:19 | Engineering design in wood | Table 4.1.8.9. 4.3.1.1.(1) A-5.1.4.1.(6)(b) and (c) A-9.15.2.4.(1) A-9.23.4.2. |
| CSA | O112.9:10 | Evaluation of adhesives for structural wood products (exterior exposure) | Table 9.10.3.1.-B |
| CSA | O112.10-08 | Evaluation of Adhesives for Structural Wood Products (Limited Moisture Exposure) | Table 9.10.3.1.-B |
| CSA | O118.1-08 | Western Red Cedar Shakes and Shingles | Table 5.9.1.1. Table 9.26.2.1.-B 9.27.7.1.(1) |
| CSA | O118.2-08 | Eastern White Cedar Shingles | Table 5.9.1.1. Table 9.26.2.1.-B 9.27.7.1.(1) |
| CSA | O121-17 | Douglas fir plywood | Table 5.9.1.1. 9.23.15.2.(1) 9.23.16.2.(1) Table 9.23.17.2.-A 9.27.8.1.(1) 9.30.2.2.(1) Table 9.23.12.3.-A Table 9.23.12.3.-B Table 9.23.12.3.-C |
| CSA | CAN/CSA-O122-16 | Structural glued-laminated timber | Table 9.23.4.2.-K Table 9.23.12.3.-D |
| CSA | CAN/CSA-O132.2 Series-90 | Wood Flush Doors | 9.7.4.3.(4) |
| CSA | O141:05 | Softwood Lumber | Table 5.9.1.1. 9.3.2.6.(1) A-9.3.2.1.(1) |
| CSA | O151-17 | Canadian softwood plywood | Table 5.9.1.1. 9.23.15.2.(1) 9.23.16.2.(1) Table 9.23.17.2.-A 9.27.8.1.(1) 9.30.2.2.(1) Table 9.23.12.3.-A Table 9.23.12.3.-B Table 9.23.12.3.-C |
| CSA | O153-13 | Poplar plywood | Table 5.9.1.1. 9.23.15.2.(1) 9.23.16.2.(1) Table 9.23.17.2.-A 9.27.8.1.(1) 9.30.2.2.(1) |
| CSA | O177-06 | Qualification Code for Manufacturers of Structural Glued-Laminated Timber | 4.3.1.2.(1) Table 9.23.4.2.-K Table 9.23.12.3.-D |
| CSA | O325-16 | Construction sheathing | Table 5.9.1.1. Table 9.23.13.6. 9.23.15.2.(1) 9.23.15.4.(2) 9.23.16.2.(1) 9.23.16.3.(2) 9.29.9.1.(2) 9.29.9.2.(5) Table 9.23.12.3.-A Table 9.23.12.3.-B Table 9.23.12.3.-C |
| CSA | O437.0-93 | OSB and Waferboard | Table 5.9.1.1. 9.23.15.2.(1) 9.23.15.4.(2) 9.23.16.2.(1) 9.23.16.3.(2) Table 9.23.17.2.-A 9.27.10.1.(1) 9.29.9.1.(2) 9.30.2.2.(1) Table 9.23.12.3.-A Table 9.23.12.3.-B Table 9.23.12.3.-C A-9.23.15.4.(2) |
| CSA | CAN/CSA-P.2-13 | Testing method for measuring the annual fuel utilization efficiency of residential gas-fired or oil-fired furnaces and boilers | Table 9.36.3.10. |
| CSA | CAN/CSA-P.3-15 | Testing method for measuring energy consumption and determining efficiencies of gas-fired and fuel oil-fired water heaters | Table 9.36.4.2. |
| CSA | CAN/CSA-P.4.1-15 | Testing method for measuring annual fireplace efficiency | Table 9.36.3.10. |
| CSA | P.6-09 | Test method for measuring thermal efficiency of gas-fired pool heaters | Table 9.36.4.2. |
| CSA | CAN/CSA-P.8-09 | Thermal efficiencies of industrial and commercial gas-fired package furnaces | Table 9.36.3.10. |
| CSA | CAN/CSA-P.9-11 | Test method for determining the performance of combined space and water heating systems (combos) | 9.36.3.10.(3) Table 9.36.3.10. Table 9.36.4.2. Table 9.36.5.15.-C |
| CSA | P.10-07 | Performance of Integrated Mechanical Systems for Residential Heating and Ventilation | 9.36.3.9.(2) Table 9.36.3.10. Table 9.36.4.2. Table 9.36.5.15.-C |
| CSA | CAN/CSA-P.11-07 | Testing Method for Measuring Efficiency and Energy Consumption of Gas-Fired Unit Heaters | Table 9.36.3.10. |
| CSA | S6-14 | Canadian Highway Bridge Design Code | A-Table 4.1.5.3. A-Table 4.1.5.9. |
| CSA | S16:19 | Design of steel structures | Table 4.1.8.9. 4.3.4.1.(1) A-4.1.5.11. A-Table 4.1.8.9. A-4.3.4.1.(1) |
| CSA | CAN/CSA-S37-18 | Antennas, towers, and antenna-supporting structures | 4.1.6.15.(1) 4.1.7.11.(1) |
| CSA | S136-16 | North American specification for the design of cold-formed steel structural members (using the Appendix B provisions applicable to Canada) | 4.1.8.1.(5) Table 4.1.8.9. 4.3.4.2.(1) |
| CSA | S157-17/S157.1-17 | Strength design in aluminum/Commentary on CSA S157-17, Strength design in aluminum | 4.3.5.1.(1) |
| CSA | S269.1-16 | Falsework and formwork | 4.1.1.3.(4) A-9.15.1.1.(1)(c) and 9.20.1.1.(1)(b) |
| CSA | S269.2-16 | Access scaffolding for construction purposes | 4.1.1.3.(4) |
| CSA | CAN/CSA-S269.3-M92 | Concrete Formwork | 4.1.1.3.(4) |
| CSA | S304-14 | Design of masonry structures | Table 4.1.8.9. 4.3.2.1.(1) A-5.1.4.1.(6)(b) and (c) |
| CSA | S367-12 | Air-, cable-, and frame-supported membrane structures | 4.4.1.1.(1) |
| CSA | S406-16 | Specification of permanent wood foundations for housing and small buildings | 9.15.2.4.(1) 9.16.5.1.(1) A-9.15.2.4.(1) |
| CSA | S413-14 | Parking structures | 4.4.2.1.(1) A-4.4.2.1.(1) |
| CSA | S478:19 | Durability in buildings | A-5.1.4.2. |
| CSA | S832:14 | Seismic risk reduction of operational and functional components (OFCs) of buildings | A-Table 4.1.8.18. |
| CSA | Z32-15 | Electrical safety and essential electrical systems in health care facilities | 3.2.7.3.(4) 3.2.7.6.(1) A-3.2.7.6.(1) |
| CSA | Z240 MH Series-16 | Manufactured homes | 1.1.1.1.(2)(4) A-1.1.1.1.(2)(4) |
| CSA | Z240.2.1-16 | Structural requirements for manufactured homes | A-1.1.1.1.(2)(4) 9.12.2.2.(6) 9.15.1.3.(1) |
| CSA | Z240.10.1:19 | Site preparation, foundation, and installation of buildings | A-1.1.1.1.(2)(4) 9.15.1.3.(1) 9.23.6.3.(1) |
| CSA | CAN/CSA-Z317.2-15 | Special requirements for heating, ventilation, and air-conditioning (HVAC) systems in health care facilities | 6.2.1.1.(1) 6.3.2.15.(6) |
| CSA | CAN/CSA-Z662-15 | Oil and gas pipeline systems | 3.2.3.22.(1) |
| CSA | Z7396.1-17 | Medical gas pipeline systems – Part 1: Pipelines for medical gases, medical vacuum, medical support gases, and anaesthetic gas scavenging systems | 3.7.3.1.(1) |
| CSSBI | 23M-2016 | Standard for Residential Steel Cladding | 9.27.11.1.(1) A-9.27.11.1.(1) |
| CWC | 1997 | Introduction to Wood Building Technology | A-9.27.3.8.(4) |
| CWC | 2000 | Wood Reference Handbook | A-9.27.3.8.(4) |
| CWC | 2009 | The Span Book | A-9.23.4.2. |
| CWC | 2014 | Engineering Guide for Wood Frame Construction | 9.4.1.1.(1) 9.23.13.1.(2) 9.23.13.2.(2) 9.23.13.3.(2) A-9.4.1.1. A-9.23.13.1. |
| DIN | EN 303-5:2012 | Heating boilers – Part 5: Heating boilers for solid fuels, manually and automatically stoked, nominal heat output of up to 500 kW – Terminology, requirements, testing and marking; German version EN 303-5:2012 | Table 9.36.3.10. |
| DOE | 10 CFR, Part 430-2011 | Energy, Energy Conservation Program for Consumer Products | Table 9.36.4.2. |
| DOE | 10 CFR, Part 431-2011 | Energy, Energy Efficiency Program for Certain Commercial and Industrial Equipment | Table 9.36.3.10. Table 9.36.4.2. |
| ECC | 2013 | EIFS Practice Manual | A-5.9.4.1.(1) A-9.27.14.1.(1) |
| EPA | 40 CFR, Part 60-2008 | Protection of Environment, Standards of Performance for New Stationary Sources | Table 9.36.3.10. |
| EPA | 625/R-92/016 (1994) | Radon Prevention in the Design and Construction of Schools and Other Large Buildings | A-5.4.1.1. 6.2.1.1.(1) |
| FEMA | 450-1-2003 | NEHRP Recommended Provisions for Seismic Regulations for New Buildings and Other Structures | A-4.1.8.18.(14) and (15) |
| FEMA | P-750-2009 | NEHRP Recommended Seismic Provisions for New Buildings and Other Structures | A-4.1.8.18.(14) and (15) |
| FLL | 2008 | Guidelines for the Planning, Construction and Maintenance of Green Roofing | A-5.6.1.2.(2) |
| FM Approvals | 2008 | Approval Standard for Quick Response Storage Sprinklers for Fire Protection | A-3.2.5.12.(10) |
| FPI | Project 43-10C-024 (1988) | Deflection Serviceability Criteria for Residential Floors | A-9.23.4.2.(2) |
| HC | 2004 | Fungal Contamination in Public Buildings: Health Effects and Investigation Methods | A-5.5.1.1. |
| HC | 2007 | Radon: A Guide for Canadian Homeowners | A-5.4.1.1. A-6.2.1.1. A-9.13.4.3. |
| HC | 2008 | Guide for Radon Measurements in Public Buildings (Schools, Hospitals, Care Facilities, Detention Centres) | A-5.4.1.1. A-6.2.1.1. |
| HC | 2008 | Guide for Radon Measurements in Residential Dwellings (Homes) | A-9.13.4.3. |
| HC | H46-2/90-156E | Exposure Guidelines for Residential Indoor Air Quality | A-6.3.1.5. A-9.25.5.2. |
| HC | R.S.C. 1985, c. H-3 | Hazardous Products Act | A-1.4.1.2.(1)(4) A-9.25.2.2.(2) |
| HC | WHMIS 1988 | Workplace Hazardous Materials Information System (WHMIS) | A-1.4.1.2.(1)(4) A-3.3.1.2.(1) |
| HC | SOR/2015-17 | Hazardous Products Regulations | 1.4.1.2.(1)(4) A-3.3.1.2.(1) |
| HPVA | ANSI/HPVA HP-1-2009 | American National Standard for Hardwood and Decorative Plywood | Table 5.9.1.1. 9.27.8.1.(1) 9.30.2.2.(1) |
| HRAI | 2017 Edition | HRAI Digest | 6.2.1.1.(1) 9.32.2.3.(4) 9.32.3.2.(1) 9.33.4.1.(1) A-9.36.3.2.(1) A-9.36.3.2.(2) A-9.36.3.4.(1) |
| HVI | HVI Publication 911 | Certified Home Ventilating Products Directory | A-9.36.3.9.(3) |
| HVI | HVI Publication 915-2013 | Loudness Testing and Rating Procedure | 9.32.3.10.(2) Table 9.32.3.10.-B |
| HVI | HVI Publication 916-2013 | Airflow Test Procedure | 9.32.3.10.(1) |
| ICC | 400-2012 | Standard on the Design and Construction of Log Structures | 9.36.2.2.(5) A-9.36.2.2.(5) |
| IEC | 60268-16:2011 | Sound system equipment – Part 16: Objective rating of speech intelligibility by speech transmission index | A-3.2.4.22.(1)(b) |
| ISO | 3864-1:2011 | Graphical symbols – Safety colours and safety signs – Part 1: Design principles for safety signs and safety markings | 3.4.5.1.(2) 9.9.11.3.(2) |
| ISO | 7010:2011 | Graphical symbols – Safety colours and safety signs – Registered safety signs | 3.4.5.1.(2) A-3.4.5.1.(2)(c) 9.9.11.3.(2) |
| ISO | 7240-19:2007 | Fire detection and alarm systems – Part 19: Design, installation, commissioning and service of sound systems for emergency purposes | A-3.2.4.22.(1)(b) |
| ISO | 7731:2003 | Ergonomics – Danger signals for public and work areas – Auditory danger signals | A-3.2.4.22.(1)(b) |
| ISO | 8201:1987 | Acoustics – Audible emergency evacuation signal | 3.2.4.18.(2) A-3.2.4.18.(2) |
| ISO | 10848-1:2006 | Acoustics – Laboratory measurement of the flanking transmission of airborne and impact sound between adjoining rooms – Part 1: Frame document | 5.8.1.4.(2) 5.8.1.4.(3) 5.8.1.5.(2) 5.8.1.5.(3) |
| ISO | 15712-1:2005 | Building acoustics - Estimation of acoustic performance of buildings from the performance of elements - Part 1: Airborne sound insulation between rooms | 5.8.1.4.(1) 5.8.1.4.(2) 5.8.1.4.(4) 5.8.1.4.(5) 5.8.1.4.(6) 5.8.1.5.(1) 5.8.1.5.(2) 5.8.1.5.(5) 5.8.1.5.(6) |
| NEMA | SB 50:2008 | Emergency Communications Audio Intelligibility Applications Guide | A-3.2.4.22.(1)(b) |
| NFPA | 2010 Edition | Fire Protection Guide to Hazardous Materials | A-6.9.1.2.(1) |
| NFPA | 2008 | Fire Protection Handbook, Twentieth Edition | A-3.2.2.2.(1) A-3.6.2.7.(5) |
| NFPA | 13-2019(6) | Standard for the Installation of Sprinkler Systems | 3.1.9.1.(4) 3.2.4.8.(2) 3.2.4.15.(1) 3.2.5.12.(1) 3.2.5.12.(9) 3.2.8.2.(5) 3.2.8.3.(2) 3.3.2.14.(3) A-3.1.11.5.(3) and (4) A-3.2.4.9.(3)(f) A-3.2.5.12.(1) A-3.2.5.12.(6) A-3.2.5.13.(1) A-3.2.8.2.(3) 9.10.9.9.(4) |
| NFPA | 13D-2016 | Standard for the Installation of Sprinkler Systems in One- and Two-Family Dwellings and Manufactured Homes | 3.2.4.1.(2) 3.2.5.12.(3) 3.2.7.9.(4) A-3.2.5.12.(2) A-3.2.5.12.(6) A-3.2.5.13.(1) 9.10.2.2.(2) 9.10.18.2.(3) |
| NFPA | 13R-2019(7) | Standard for the Installation of Sprinkler Systems in Low-Rise Residential Occupancies | 3.2.5.12.(2) A-3.2.5.12.(2) A-3.2.5.12.(6) A-3.2.5.13.(1) |
| NFPA | 14-2013 | Standard for the Installation of Standpipe and Hose Systems | 3.2.5.9.(1) 3.2.5.10.(1) |
| NFPA | 20-2016 | Standard for the Installation of Stationary Pumps for Fire Protection | 3.2.4.9.(4) 3.2.5.18.(1) A-3.2.4.9.(3)(f) |
| NFPA | 30-2018 | Flammable and Combustible Liquids Code | A-6.9.1.2.(1) |
| NFPA | 30A-2018 | Code for Motor Fuel Dispensing Facilities and Repair Garages | A-6.9.1.2.(1) |
| NFPA | 32-2016 | Standard for Drycleaning Facilities | A-6.9.1.2.(1) |
| NFPA | 33-2018 | Standard for Spray Application Using Flammable or Combustible Materials | A-6.9.1.2.(1) |
| NFPA | 34-2018 | Standard for Dipping, Coating, and Printing Processes Using Flammable or Combustible Liquids | A-6.9.1.2.(1) |
| NFPA | 35-2016 | Standard for Manufacture of Organic Coatings | A-6.9.1.2.(1) |
| NFPA | 36-2017 | Standard for Solvent Extraction Plants | A-6.9.1.2.(1) |
| NFPA | 40-2019 | Standard for the Storage and Handling of Cellulose Nitrate Film | A-6.9.1.2.(1) |
| NFPA | 51-2018 | Standard for the Design and Installation of Oxygen-Fuel Gas Systems for Welding, Cutting, and Allied Processes | A-6.9.1.2.(1) |
| NFPA | 51A-2012 | Standard for Acetylene Cylinder Charging Plants | A-6.9.1.2.(1) |
| NFPA | 55-2020 | Compressed Gases and Cryogenic Fluids Code | A-6.9.1.2.(1) |
| NFPA | 61-2017 | Standard for the Prevention of Fires and Dust Explosions in Agricultural and Food Processing Facilities | A-6.9.1.2.(1) |
| NFPA | 68-2013 | Standard on Explosion Protection by Deflagration Venting | 3.3.6.4.(2) A-3.6.2.7.(5) A-6.9.1.2.(1) |
| NFPA | 69-2014 | Standard on Explosion Prevention Systems | A-3.6.2.7.(5) A-6.9.1.2.(1) |
| NFPA | 72-2019 | National Fire Alarm and Signaling Code | A-3.2.4.22.(1)(b) |
| NFPA | 80-2013 | Standard for Fire Doors and Other Opening Protectives | 3.1.8.5.(2) 3.1.8.12.(2) 3.1.8.16.(1) 3.1.9.1.(5) A-3.1.8.1.(2) A-3.2.8.2.(3) 9.10.9.9.(5) 9.10.13.1.(1) |
| NFPA | 80A-2012 | Recommended Practice for Protection of Buildings from Exterior Fire Exposures | A-3 |
| NFPA | 82-2014 | Standard on Incinerators and Waste and Linen Handling Systems and Equipment | 6.2.2.1.(1) 9.10.10.5.(2) |
| NFPA | 85-2019 | Boiler and Combustion Systems Hazards Code | A-6.9.1.2.(1) |
| NFPA | 86-2019 | Standard for Ovens and Furnaces | A-6.9.1.2.(1) |
| NFPA | 88A-2019 | Standard for Parking Structures | A-6.9.1.2.(1) |
| NFPA | 91-2015 | Standard for Exhaust Systems for Air Conveying of Vapors, Gases, Mists, and Particulate Solids | 6.3.4.3.(1) A-6.9.1.2.(1) |
| NFPA | 96-2014 | Standard for Ventilation Control and Fire Protection of Commercial Cooking Operations | 3.2.4.8.(2) 3.6.3.5.(1) A-3.3.1.2.(2) A-3.6.3.5. 6.3.1.6.(1) A-6.9.1.2.(1) A-9.10.1.4.(1) |
| NFPA | 101-2018 | Life Safety Code | 3.3.2.1.(2) 3.3.2.1.(3) A-3.3.2.1.(2) |
| NFPA | 105-2013 | Standard for Smoke Door Assemblies and Other Opening Protectives | 3.1.8.5.(3) 3.1.8.5.(7) |
| NFPA | 204-2018 | Standard for Smoke and Heat Venting | A-6.9.1.2.(1) |
| NFPA | 211-2019 | Standard for Chimneys, Fireplaces, Vents, and Solid Fuel-Burning Appliances | 6.3.3.2.(2) 6.3.3.3.(1) |
| NFPA | 303-2016 | Fire Protection Standard for Marinas and Boatyards | A-6.9.1.2.(1) |
| NFPA | 307-2016 | Standard for the Construction and Fire Protection of Marine Terminals, Piers, and Wharves | A-6.9.1.2.(1) |
| NFPA | 409-2016 | Standard on Aircraft Hangars | A-6.9.1.2.(1) |
| NFPA | 415-2016 | Standard on Airport Terminal Buildings, Fueling Ramp Drainage, and Loading Walkways | A-6.9.1.2.(1) |
| NFPA | 484-2019 | Standard for Combustible Metals | A-6.9.1.2.(1) |
| NFPA | 654-2017 | Standard for the Prevention of Fire and Dust Explosions from the Manufacturing, Processing, and Handling of Combustible Particulate Solids | A-6.9.1.2.(1) |
| NFPA | 655-2017 | Standard for Prevention of Sulfur Fires and Explosions | A-6.9.1.2.(1) |
| NFPA | 664-2017 | Standard for the Prevention of Fires and Explosions in Wood Processing and Woodworking Facilities | A-6.9.1.2.(1) |
| NFPA | 1142-2007 | Standard on Water Supplies for Suburban and Rural Fire Fighting | A-3.2.5.7.(1) |
| NFPA | 1710-2010 | Standard for the Organization and Deployment of Fire Suppression Operations, Emergency Medical Operations, and Special Operations to the Public by Career Fire Departments | A-3.2.3.1.(8) |
| NFRC | 100-2010 | Procedure for Determining Fenestration Product U-factors | 9.36.2.2.(3) |
| NFRC | 200-2010 | Procedure for Determining Fenestration Product Solar Heat Gain Coefficient and Visible Transmittance at Normal Incidence | 9.36.2.2.(3) |
| NLGA | 2017 | Standard Grading Rules for Canadian Lumber | 9.3.2.1.(1) A-9.3.2.1.(1) Table A-9.3.2.1.(1)-A A-Table 9.3.2.1. A-9.3.2.8.(1) A-9.23.10.4.(1) |
| NLGA | SPS-1-2017 | Fingerjoined Structural Lumber | Table 9.10.3.1.-A A-9.23.10.4.(1) |
| NLGA | SPS-3-2017 | Fingerjoined “Vertical Stud Use Only” Lumber | Table 9.10.3.1.-A A-9.23.10.4.(1) |
| NRC | 1988 | Performance and acceptability of wood floors – Forintek studies | A-9.23.4.2.(2) |
| NRC | 2005 | A Guide for the Wind Design of Mechanically Attached Flexible Membrane Roofs | A-5.2.2.2.(4) |
| NRC | 17808-2005 | Performance Guidelines for Basement Envelope Systems and Materials: Final Research Report | A-9.25.5.1. |
| NRC | BPN 54-85 | The difference between a vapour barrier and an air barrier | A-9.25.1.1.(2) |
| NRC | CBD 222 | Airtight houses and carbon monoxide poisoning | A-9.33.1.1.(2) |
| NRC | CBD 230 | Applying building codes to existing buildings | A-1.1.1.1.(1)(4) |
| NRC | CBD 231 | Moisture problems in houses | A-9.25.3.1.(1) |
| NRC | CRBCPI-Y2-R19 | Guideline on Design for Durability of Building Envelopes | A-5.1.4.2. A-5.4.1.1.(3) |
| NRC | NRCC 49677-2007 | Best Practice Guide on Fire Stops and Fire Blocks and their Impact on Sound Transmission | A-9.11. |
| NRC | RR-331-2017 | Guide to Calculating Airborne Sound Transmission in Buildings | A-5.8.1.4. A-5.8.1.4.(4)(b) |
| NRCA | 3rd Edition, 2017 | The NRCA Vegetative Roof Systems Manual | A-5.6.1.2.(2) |
| NRCan | R.S.C. 1985, c. E-17 | Explosives Act | 3.3.6.2.(3) |
| NRCan | SOR/2016-311 | Energy Efficiency Regulations, 2016 | Table 9.36.4.2. |
| NYCDH | 2008 | Guidelines on Assessment and Remediation of Fungi in Indoor Environments | A-5.5.1.1. |
| OMMAH | 2012 | 2012 Building Code Compendium, Volume 2, Supplementary Standard SB-7, Guards for Housing and Small Buildings | A-9.8.8.2. |
| SMACNA | ANSI/SMACNA 006-2006 | HVAC Duct Construction Standards – Metal and Flexible | 9.33.6.5.(2) A-9.36.3.2.(2) |
| SPRI | ANSI/GRHC/SPRI VR-1-2018 | Procedure for Investigating Resistance to Root or Rhizome Penetration on Vegetative Roofs | 5.6.1.2.(2) |
| SPRI | ANSI/SPRI WD-1-2014 | Wind Design Standard Practice for Roofing Assemblies | A-5.2.2.2.(4) |
| TC | SOR/96-433 | Canadian Aviation Regulations – Part III | 4.1.5.13.(1) |
| TC | SOR/2001-286 | Transportation of Dangerous Goods Regulations (TDGR) | 1.4.1.2.(1)(4) A-1.4.1.2.(1)(4) A-3.3.1.2.(1) |
| TIAC | 2013 | Mechanical Insulation Best Practices Guide | A-6.3.2.5. |
| TPIC | 2019 | Truss Design Procedures and Specifications for Light Metal Plate Connected Wood Trusses | 9.23.14.11.(1) |
| TWC | 1993 | Details of Air Barrier Systems for Houses | Table A-9.25.5.1.(1) |
| UL | ANSI/CAN/UL/ULC 300-2019 | Standard for Fire Testing of Fire Extinguishing Systems for Protection of Commercial Cooking Equipment | 6.9.1.3.(1) |
| UL | ANSI/UL 1784-2015 | Standard for Air Leakage Tests of Door Assemblies and Other Opening Protectives | 3.1.8.4.(4) |
| UL | 2034-2008 | Standard for Single and Multiple Station Carbon Monoxide Alarms | 6.9.3.1.(6) 9.32.4.2.(8) |
| ULC | CAN/ULC-S101-14 | Standard Method of Fire Endurance Tests of Building Construction and Materials | 3.1.5.7.(2) 3.1.5.14.(5) 3.1.5.14.(6) 3.1.5.15.(3) 3.1.5.15.(4) 3.1.7.1.(1) 3.1.11.7.(1) 3.2.3.8.(1) A-3.1.5.14.(5)(d) 9.10.16.3.(1) Table 9.10.3.1.-B |
| ULC | CAN/ULC-S102-10 | Standard Method of Test for Surface Burning Characteristics of Building Materials and Assemblies | 3.1.5.24.(1) 3.1.12.1.(1) Table 5.9.1.1. Table 9.23.17.2.-A 9.29.5.2.(1) |
| ULC | CAN/ULC-S102.2:2018 | Standard Method of Test for Surface Burning Characteristics of Flooring, Floor Coverings, and Miscellaneous Materials and Assemblies | 3.1.12.1.(2) 3.1.13.4.(1) 9.27.12.1.(4) 9.27.13.1.(2) |
| ULC | CAN/ULC-S102.3:2018 | Standard Method of Fire Test of Light Diffusers and Lenses | 3.1.13.4.(1) |
| ULC | CAN/ULC-S102.4:2017 | Standard Method of Test for Fire and Smoke Characteristics of Electrical Wiring, Cables and Non-Metallic Raceways | 3.1.4.3.(2) 3.1.5.21.(2) 3.1.5.23.(2) |
| ULC | CAN/ULC-S104-15 | Standard Method for Fire Tests of Door Assemblies | 3.1.8.4.(1) 3.2.6.5.(3) |
| ULC | CAN/ULC-S105:2016 | Standard Specification for Fire Door Frames Meeting the Performance Required by CAN/ULC-S104 | 9.10.13.6.(1) |
| ULC | CAN/ULC-S106-15 | Standard Method for Fire Tests of Window and Glass Block Assemblies | 3.1.8.4.(1) |
| ULC | CAN/ULC-S107:2019 | Standard Methods of Fire Tests of Roof Coverings | 3.1.15.1.(1) |
| ULC | CAN/ULC-S109-14 | Standard Method for Flame Tests of Flame-Resistant Fabrics and Films | 3.1.16.1.(1) 3.1.18.5.(1) 3.6.5.2.(2) 3.6.5.3.(1) 9.33.6.3.(1) |
| ULC | CAN/ULC-S110-13 | Standard Methods of Test for Air Ducts | 3.6.5.1.(2) 3.6.5.1.(5) 9.33.6.2.(2) 9.33.6.2.(4) |
| ULC | CAN/ULC-S111-13 | Standard Method of Fire Tests for Air Filter Units | 6.3.2.13.(1) 9.33.6.14.(1) |
| ULC | CAN/ULC-S112-10 | Standard Method of Fire Test of Fire Damper Assemblies | 3.1.8.4.(1) A-3.2.6.6.(1) |
| ULC | CAN/ULC-S112.1-10 | Standard for Leakage Rated Dampers for Use in Smoke Control Systems | 3.1.8.4.(3) 6.3.2.7.(3) |
| ULC | CAN/ULC-S112.2-07 | Standard Method of Fire Test of Ceiling Firestop Flap Assemblies | 3.6.4.3.(2) 9.10.13.14.(1) |
| ULC | CAN/ULC-S113:2016 | Standard Specification for Wood Core Doors Meeting the Performance Required by CAN/ULC-S104 for Twenty Minute Fire Rated Closure Assemblies | 9.10.13.2.(1) A-9.10.9.3.(2) A-9.10.13.2.(1) |
| ULC | CAN/ULC-S114:2018 | Standard Method of Test for Determination of Non-Combustibility in Building Materials | 1.4.1.2.(1)(4) |
| ULC | CAN/ULC-S115-11 | Standard Method of Fire Tests of Firestop Systems | 3.1.5.19.(3) 3.1.8.3.(3) 3.1.9.1.(1) 3.1.9.1.(2) 3.1.9.1.(3) 3.1.9.1.(6) 3.1.9.1.(7) 3.1.9.3.(1) 3.1.9.3.(2) 3.1.9.3.(4) 3.1.9.4.(4) 3.1.9.4.(7) A-3.1.8.3.(2) A-3.1.11.7.(7) 9.10.9.2.(3) 9.10.9.6.(1) 9.10.9.6.(2) 9.10.9.8.(1) 9.10.9.8.(6) |
| ULC | CAN/ULC-S124-06 | Standard Method of Test for the Evaluation of Protective Coverings for Foamed Plastic | 3.1.5.15.(2) A-3.1.5.14.(5)(d) |
| ULC | CAN/ULC-S126-14 | Standard Method of Test for Fire Spread Under Roof-Deck Assemblies | 3.1.14.1.(1) 3.1.14.2.(1) |
| ULC | CAN/ULC-S134-13 | Standard Method of Fire Test of Exterior Wall Assemblies | 3.1.5.5.(1) 9.10.14.5.(2) 9.10.15.5.(2) 9.10.15.5.(3) |
| ULC | ULC-S135-04 | Standard Test Method for the Determination of Combustibility Parameters of Building Materials Using an Oxygen Consumption Calorimeter (Cone Calorimeter) | 3.1.5.1.(2) |
| ULC | CAN/ULC-S138-06 | Standard Method of Test for Fire Growth of Insulated Building Panels in a Full-Scale Room Configuration | 3.1.5.7.(1) 3.1.5.7.(3) |
| ULC | CAN/ULC-S139:2017 | Standard for Fire Test for Circuit Integrity of Fire-Resistive Power, Instrumentation, Control and Data Cables | 3.2.6.5.(6) 3.2.7.10.(2) 3.2.7.10.(3) |
| ULC | CAN/ULC-S143-14 | Standard Method of Fire Tests for Non-Metallic Electrical and Optical Fibre Cable Raceway Systems | 3.1.5.23.(1) |
| ULC | CAN/ULC-S144-12 | Standard Method of Fire Resistance Test – Grease Duct Assemblies | 3.6.3.5.(2) A-3.6.3.5. |
| ULC | CAN/ULC-S146-19 | Standard Method of Test for the Evaluation of Encapsulation Materials and Assemblies of Materials for the Protection of Structural Timber Elements | 3.1.6.5.(1) |
| ULC | ULC-S332-93 | Standard for Burglary Resisting Glazing Material | A-9.7.5.2.(1) |
| ULC | ULC-S505-74 | Standard for Fusible Links for Fire Protection Services | 3.1.8.10.(2) |
| ULC | CAN/ULC-S524:2019 | Standard for Installation of Fire Alarm Systems | 3.1.8.11.(3) 3.1.8.14.(3) 3.2.4.5.(1) 3.2.4.19.(5) 3.2.4.20.(7) 3.2.4.20.(8) 3.2.4.20.(10) 3.2.4.20.(15) A-3.2.4.7.(4) A-3.2.4.18.(9) and (10) A-3.2.4.19.(1)(g) A-3.2.4.20.(10) 9.10.19.4.(3) 9.10.19.6.(2) |
| ULC | CAN/ULC-S526-2016 | Visible Signaling Devices for Fire Alarm and Signaling Systems, Including Accessories | 3.2.4.19.(5) 3.2.4.19.(6) A-3.2.4.19.(3) |
| ULC | CAN/ULC-S531:2019 | Standard for Smoke Alarms | 3.2.4.20.(2) 3.3.2.18.(4) 9.10.19.1.(1) |
| ULC | CAN/ULC-S537:2019 | Standard for Verification of Fire Alarm Systems | 3.2.4.5.(2) |
| ULC | CAN/ULC-S540-13 | Standard for Residential Fire and Life Safety Warning Systems: Installation, Inspection, Testing and Maintenance | 3.2.4.21.(1) 9.10.2.2.(3) 9.10.2.2.(4) 9.10.19.8.(1) |
| ULC | CAN/ULC-S553-14 | Standard for the Installation of Smoke Alarms | 3.2.4.20.(13) 9.10.19.3.(2) |
| ULC | CAN/ULC-S561-13 | Standard for Installation and Services for Fire Signal Receiving Centres and Systems | 3.2.4.7.(4) A-3.2.4.7.(4) |
| ULC | CAN/ULC-S572:2017 | Standard for Photoluminescent and Self-Luminous Exit Signs and Path Marking Systems | 3.4.5.1.(3) 3.4.5.1.(4) A-3.4.5.1.(4) 9.9.11.3.(3) 9.9.11.3.(4) |
| ULC | CAN/ULC-S610:2018 | Standard for Factory-Built Fireplace Systems | 9.22.8.1.(1) |
| ULC | ULC-S628-93 | Standard for Fireplace Inserts | 9.22.10.1.(1) |
| ULC | CAN/ULC-S629:2016 | Standard for 650°C Factory-Built Chimneys | 9.33.10.2.(1) |
| ULC | CAN/ULC-S639-M87 | Standard for Steel Liner Assemblies for Solid-Fuel Burning Masonry Fireplaces | 9.22.2.3.(1) |
| ULC | CAN/ULC-S701.1:2017 | Standard for Thermal Insulation, Polystyrene Boards | Table 5.9.1.1. Table 9.23.17.2.-A 9.25.2.2.(1) Table A-9.36.2.4.(1)-D |
| ULC | CAN/ULC-S702.1-14 | Standard for Mineral Fibre Thermal Insulation for Buildings, Part 1: Material Specification | 3.1.6.3.(4) Table 5.9.1.1. A-5.9.1.1.(1) 9.10.9.8.(3) 9.10.3.1.(3) Table 9.23.17.2.-A 9.25.2.2.(1) Table A-9.36.2.4.(1)-D |
| ULC | CAN/ULC-S703-09 | Standard for Cellulose Fibre Insulation (CFI) for Buildings | Table 5.9.1.1. 9.10.3.1.(3) 9.25.2.2.(1) Table A-9.36.2.4.(1)-D |
| ULC | CAN/ULC-S704.1:2017 | Standard for Thermal Insulation, Polyurethane and Polyisocyanurate, Boards, Faced | Table 5.9.1.1. Table 9.23.17.2.-A 9.25.2.2.(1) Table A-9.36.2.4.(1)-D |
| ULC | CAN/ULC-S705.1-18 | Standard for Thermal Insulation – Spray Applied Rigid Polyurethane Foam, Medium Density – Material Specification | Table 5.9.1.1. 9.25.2.2.(1) Table A-9.36.2.4.(1)-D |
| ULC | CAN/ULC-S705.2-05 | Standard for Thermal Insulation – Spray Applied Rigid Polyurethane Foam, Medium Density – Application | Table 5.9.1.1. 9.25.2.5.(1) |
| ULC | CAN/ULC-S706.1:2016 | Standard for Wood Fibre Insulating Boards for Buildings | Table 5.9.1.1. 9.23.16.7.(3) Table 9.23.17.2.-A 9.25.2.2.(1) 9.29.8.1.(1) |
| ULC | CAN/ULC-S710.1:2019 | Standard for Bead-Applied One Component Polyurethane Air Sealant Foam, Part 1: Material Specification | Table 5.9.1.1. 9.36.2.10.(6) |
| ULC | CAN/ULC-S711.1:2019 | Standard for Bead-Applied Two Component Polyurethane Air Sealant Foam, Part 1: Material Specification | Table 5.9.1.1. 9.36.2.10.(6) |
| ULC | CAN/ULC-S712.1:2017 | Standard for Thermal Insulation - Light Density, Open Cell Spray Applied Semi-Rigid Polyurethane Foam - Material Specification | Table A-9.36.2.4.(1)-D |
| ULC | CAN/ULC-S716.1-12 | Standard for Exterior Insulation and Finish Systems (EIFS) - Materials and Systems | 5.9.4.1.(1) A-5.9.4.1.(1) 9.27.14.1.(1) 9.27.14.2.(1) A-9.27.14.2.(2)(a) |
| ULC | CAN/ULC-S716.2-12 | Standard for Exterior Insulation and Finish Systems (EIFS) - Installation of EIFS Components and Water Resistive Barrier | A-5.9.4.1.(1) 9.27.14.3.(1) |
| ULC | CAN/ULC-S716.3-12 | Standard for Exterior Insulation and Finish System (EIFS) - Design Application | A-5.9.4.1.(1) 9.27.14.3.(1) |
| ULC | CAN/ULC-S717.1:2017 | Standard for Flat Wall Insulating Concrete Form (ICF) Units – Material Properties | Table 5.9.1.1. 9.15.4.1.(1) |
| ULC | CAN/ULC-S741-08 | Standard for Air Barrier Materials – Specification | 5.4.1.2.(2) 9.36.2.10.(1) |
| ULC | CAN/ULC-S742-11 | Standard for Air Barrier Assemblies – Specification | 5.4.1.2.(1) 5.4.1.2.(2) A-5.4.1.1.(3) A-5.4.1.2.(1) A-5.4.1.2.(2) A-5.4.1.2.(4) 9.36.2.9.(1) A-9.36.2.9.(1) A-9.36.2.10.(5)(b) |
| ULC | CAN/ULC-S770-15 | Standard Test Method for Determination of Long-Term Thermal Resistance of Closed-Cell Thermal Insulating Foams | Table A-9.36.2.4.(1)-D |
| ULC | CAN/ULC-S1001-11 | Standard for Integrated Systems Testing of Fire Protection and Life Safety Systems | 3.2.9.1.(1) A-3.2.9.1.(1) 9.10.1.2.(1) |
| ULC | ULC/ORD-C199P-02 | Combustible Piping for Sprinkler Systems | 3.2.5.13.(2) 3.2.5.13.(5) |
| ULC | ULC/ORD-C1254.6-95 | Fire Testing of Restaurant Cooking Area Fire Extinguishing System Units | 6.9.1.3.(1) |
| USACE | Version 3-2012 | Air Leakage Test Protocol for Building Envelope | 10.2.3.5.(1) |
| U.S. Congress |  | National Appliance Energy Conservation Act of 1987 | Table 9.36.4.2. Table 9.36.5.16. |
| WCLIB | No. 17 (2004) | Grading Rules for West Coast Lumber | A-Table 9.3.2.1. |
| WWPA | 2017 | Western Lumber Grading Rules 2017 | A-Table 9.3.2.1. |

Notes to Table 1.3.1.2.:

(1) While every effort was made to ensure the accuracy of the information in this Table, the NRC is not responsible for the accuracy, timeliness or reliability of the content presented therein. For all purposes of interpreting and applying the referenced standards, Code users should refer to the most recent official versions of the referenced editions.

(2) See Table D-1.1.2. of Appendix D for the list of standards referenced therein.

(3) Some documents may have been reaffirmed or reapproved. Check with the applicable issuing agency for up-to-date information.

(4) Code reference is in Division A.

(5) Code reference is in Division C.

(6) Subsection 9.3.15, Sprinkler-Protected Glazing, does not apply in the context of Division B.

(7) Subsection 6.5.3, Sprinkler-Protected Glazing, does not apply in the context of Division B.

1.3.2. Organizations

1.3.2.1. Abbreviations of Proper Names

1)   The abbreviations of proper names in this Code shall have the meanings assigned to them in this Article.

AAMA    Fenestration and Glazing Industry Alliance (formerly American Architectural Manufacturers Association) ( www.fgiaonline.org)

ACGIH    American Conference of Governmental Industrial Hygienists ( www.acgih.org)

ACI    American Concrete Institute ( www.concrete.org)

AHRI    Air-Conditioning, Heating and Refrigeration Institute ( www.ahrinet.org)

AISI    American Iron and Steel Institute ( www.steel.org)

ANSI    American National Standards Institute ( www.ansi.org)

APA    APA — The Engineered Wood Association ( www.apawood.org)

ASCE    American Society of Civil Engineers ( www.asce.org)

ASHRAE    American Society of Heating, Refrigerating and Air-Conditioning Engineers ( www.ashrae.org)

ASME    American Society of Mechanical Engineers ( www.asme.org)

ASTM    ASTM International ( www.astm.org)

BNQ    Bureau de normalisation du Québec ( www.bnq.qc.ca/en)

CAN    National Standard of Canada designation ( www.scc.ca) (The number or name following the CAN designation represents the agency under whose auspices the standard is issued.)

CAN3 designates CSA

CCBFC    Canadian Commission on Building and Fire Codes (see NRC)

CCME    Canadian Council of Ministers of the Environment ( www.ccme.ca)

CGSB    Canadian General Standards Board 
(www.tpsgc-pwgsc.gc.ca/ongc-cgsb/index-eng.html)

CHC    Canadian Hydronics Council ( www.chchydro.com)

CISC    Canadian Institute of Steel Construction ( www.cisc-icca.ca)

CMHC    Canada Mortgage and Housing Corporation ( www.cmhc.ca)

CoV    City of Vancouver (www.vancouver.ca)

CRCA    Canadian Roofing Contractors' Association ( www.roofingcanada.com)

CSA    CSA Group ( www.csagroup.org)

CSSBI    Canadian Sheet Steel Building Institute ( www.cssbi.ca)

CWC    Canadian Wood Council ( www.cwc.ca)

DOE    U.S. Department of Energy ( www.energy.gov)

EC    Environment and Climate Change Canada ( www.ec.gc.ca)

ECC    EIFS Council of Canada ( www.eifscouncil.org)

EPA    Environmental Protection Agency (U.S.) ( www.epa.gov)

FEMA    Federal Emergency Management Agency (U.S.) ( www.fema.gov)

FLL    German Landscape Research, Development and Construction Society ( shop.fll.de/en)

FPI    FPInnovations – Wood Products (formerly FCC – Forintek Canada Corporation) ( www.fpinnovations.ca)

GRHC    Green Roofs for Healthy Cities ( www.greenroofs.org)

HC    Health Canada ( www.hc-sc.gc.ca)

HPVA    Decorative Hardwoods Association (formerly Hardwood Plywood & Veneer Association) ( www.decorativehardwoods.org)

HRAI    Heating, Refrigeration and Air Conditioning Institute of Canada ( www.hrai.ca)

HVI    Home Ventilating Institute ( www.hvi.org)

ICC    International Code Council ( www.iccsafe.org)

IEC    International Electrotechnical Commission ( www.iec.ch)

ISO    International Organization for Standardization ( www.iso.org)

NBC    National Building Code of Canada 2020

NCMA    National Concrete Masonry Association ( www.ncma.org)

NECB    National Energy Code of Canada for Buildings 2020

NEMA    National Electrical Manufacturers Association ( www.nema.org)

NFC    National Fire Code of Canada 2020

NFPA    National Fire Protection Association ( www.nfpa.org)

NFRC    National Fenestration Rating Council ( www.nfrc.org)

NLGA    National Lumber Grades Authority ( www.nlga.org)

NPC    National Plumbing Code of Canada 2020

NRC    National Research Council of Canada ( nrc.canada.ca)

NRCA    National Roofing Contractors Association ( www.nrca.net)

NRCan    Natural Resources Canada ( www.nrcan.gc.ca)

NYCDH    New York City Department of Health and Mental Hygiene ( www.nyc.gov/health)

OMMAH    Ontario Ministry of Municipal Affairs and Housing ( www.mah.gov.on.ca)

SEI    Structural Engineering Institute 
( www.asce.org/structural-engineering/structural-engineering)

SMACNA    Sheet Metal and Air Conditioning Contractors' National Association ( www.smacna.org)

SPRI    Single Ply Roofing Industry ( www.spri.org)

TC    Transport Canada ( tc.canada.ca)

TECA    Thermal Environmental Comfort Association (www.teca.ca)

TIAC    Thermal Insulation Association of Canada ( www.tiac.ca)

TPIC    Truss Plate Institute of Canada ( www.tpic.ca)

TWC    Tarion Warranty Corporation (formerly Ontario New Home Warranty Program) ( www.tarion.com)

UL    Underwriters' Laboratories Inc. ( www.ul.com)

ULC    ULC Standards ( canada.ul.com/ulcstandards)

USACE    United States Army Corps of Engineers (www.erdc.usace.army.mil/Locations/CERL)

WCLIB    Pacific Lumber Inspection Bureau (formerly West Coast Lumber Inspection Bureau) ( www.plib.org)

WWPA    Western Wood Products Association ( www.wwpa.org)

Notes to Part 1

General

A-1.1.2.1.(1)   Objectives and Functional Statements Attributed to Acceptable Solutions. The objectives and functional statements attributed to each Code provision are listed in a table following the provisions in each Part.

Many provisions in Division B serve as modifiers of or pointers to other provisions, or serve other clarification or explanatory purposes. In most cases, no objectives and functional statements have been attributed to such provisions, which therefore do not appear in the above-mentioned tables.

For provisions that serve as modifiers of or pointers to other referenced provisions and that do not have any objectives and functional statements attributed to them, the objectives and functional statements that should be used are those attributed to the provisions they reference.

A-1.1.3.1.(2)   Climatic Values. Climatic values for municipalities not listed in Appendix C can be obtained at www.climate.weather.gc.ca or by e-mail from the Engineering Climate Services Unit of Environment and Climate Change Canada at scg-ecs@ec.gc.ca.

A-1.1.3.1.(3)   Winter Design Temperatures. The 2.5% values referred to in Sentence 1.1.3.1.(3) are the least restrictive temperatures that can be used. A designer may choose to use the 1% values given in Appendix C, which are in excess of the Code minimums but are considered acceptable.

A-1.1.3.1.(4)   Seismic Values. Figure A-1.1.3.1.(4) illustrates how to determine the seismic hazard values to be used in the application of the Part 4 and Part 9 seismic provisions.

Figure A-1.1.3.1.(4)

Determining seismic hazard values for use in Part 4 and Part 9

Notes to Figure A-1.1.3.1.(4):

(1) The abbreviations used in the figure have the following meanings:

AHJ = authority having jurisdiction

NPARC = NRC Publications Archive

(2) See also the section entitled “Seismic Hazard for Part 4” in Appendix C.

(3) See also the section entitled “Seismic Hazard for Part 9” in Appendix C.

(4) The seismic hazard values available on NPARC at https://doi.org/10.4224/nqzr-dz38 were generated from the 2020 National Building Code of Canada Seismic Hazard Tool. This subset of values on NPARC is provided as a static, archival record for Code users.

(5) The 2020 National Building Code of Canada Seismic Hazard Tool is available at https://doi.org/10.23687/b1bd3cf0-0672-47f4-8bfa-290ae75fde9b.

(6) Refer to the procedure set out in the section entitled “Seismic Hazard for Part 9” in Appendix C.